Karol Bunkowski

kbunkow@fuw.edu.pl

Warsaw University

TTC – useful information

- Even if B-Go FIFO is not loaded, but if the b-go mode is set in the way that something can be sent (for example enable=1, sync=0, single=1, FIFO =1 and retransmit =0) the “0” broadcast is sent and the BrcStr1 and BrcStr2 on TTCrx are active. So if you want to avoid this, for B-Go FIFO that you don’t want to use set all mode selection bits to “0”.

- Always connect TTCvx output clock with TTCvi input clock!!!!

- After L1A signal there are sent four ”long format asynchronous cycles” containing trigger type and Even or Orbit Count (see page 7 of TTCvi manual). In new TTCvi MKII you can set some of bits of this broadcast. In particular you can set bit “i/e” that says if the data should be received by external bus (i/e  = 1) or internal TTCrx register (i/e  = 0). But it is nonsense to set this bit to “0” since you don’t have control on address and data bits of this broadcast and it would change TTCrx control registers.

- If want to use more then one B-Go FIFO you should set the Inhibit signals duration or delay so as the time interval between ends of two Inhibit signals was bigger then time of transmission of long or short cycle (> 24 bx for short and > 40 bx for long format cycle). Transmission of the command commences at the end of the Inhibit signal.

- A design flaw has been spotted by a user, which consist in multiple -channel transfers during one B-Go<n> generation. This problem occurs when one is using the front panel input and the active part of the B-Go signal is longer than the B-channel transfer duration.  This is due to the B-Go inputs being level sensitive.  If you use a B-Go input pulse being < 400 ns you will not see this problem. If this would cause a problem, please, contact me and I will arrange for having the BCHREQ FPGA reprogrammed (Per Gällnö). (so you can’t use the orbit signal from front panel output as a B-Go signal, because it is to long – K.B.)