CLOCK  AND  CONTROL  BOARD  FOR THE  CSC EMU  PERIPHERAL AND TRACK  FINDER  ELECTRONICS

CCB2004   Specification (Production Board)

Rice University

Version 3.4.5

October 21, 2010

# Abstract

This document describes the functionality of the production version of the  Clock and Control  Board (CCB2004) for the CMS EMU peripheral electronics and its communication with other modules  in the peripheral crate.  This CCB2004 can also be used in the CSC Track Finder crate.

# Introduction

The CSC EMU electronics includes chamber-mounted Front End Boards (FEB), connected to electronics on the periphery of the detector. The peripheral Trigger/DAQ electronics  resides in VME 9U crates and includes: the combined Cathode LCT/Trigger Motherboard cards (TMB), the Data Acquisition Motherboard  (DAQMB), the Muon Port Card (MPC) and the  Clock and Control Board (CCB).  More details about the peripheral backplane and communications between modules are given in [1].

All elements of the EMU electronics will be synchronized with the LHC bunch crossing frequency. The Timing, Trigger and Control (TTC) system, which is used for this purpose for all LHC detectors, has been specified and a detailed description of this system and its functionality can be found, for example, in [2-3].  The TTC system is based on an optical fan-out system and provides for the distribution of the LHC  timing reference signal, the first level trigger decisions and control commands to about 1000 destinations.  At  the lowest level of the TTC system,  the TTCrx ASIC [3] receives the control and synchronization data from the central TTC system through the optical cable and  outputs  TTL-compatible TTC signals in parallel form.  The TTCrx is mounted on a small mezzanine card that is designed at CERN and installed on the main Clock and Control Board.

Block diagram of the  CCB2004 is shown on Figure 1. There are several new features added into design as compared to the previous version which we will refer to as CCB’2001 [4]. The new CCB2004 board can accommodate not only the TTCrm [3] mezzanine with the TTCrx ASIC, but also the most recent version of the TTC mezzanine card, the TTCrq [5], that has an additional 26-pin connector and provides low-jitter clock outputs of  40Mhz, 80Mhz and 160MHz.  The CCB2004 may decode several commands not only from the broadcast lines Brcst[7..2] as the CCB2001, but also from the Data[7..0] bus that is used for the individually addressed  commands.

The CCB2001 functionality was implemented using a mezzanine mounted Altera EPF10K100A PLD. In order to reduce the cost, improve radiation tolerance and decrease the reconfiguration time we decided to use a Xilinx Virtex-2 FPGA mounted  directly on the main CCB2004 board. The most critical part of the CCB2004 functionality is  implemented in discrete CMOS logic that is immune to single-event upsets (SEU) at the expected dosage. These functions include the VME address and command decoding, generation of Hard_reset commands, clock and command distribution to custom backplane. An FPGA may experience SEU’s however, and in order to prevent malfunctioning, it can be reloaded   from its EPROM upon common or CCB-specific Hard_reset command.

Another new feature is the ability to distribute an 80Mhz clock from the TTRrq  to the Muon Port Card over dedicated LVDS lines. This low jitter clock can be used to drive the gigabit serializers residing on the MPC board.

Estimated power consumption: +3.3V - < 2 A, +5V - <1 A.

Figure 1:  Block Diagram of the CCB’2004

- Modes of Operations

The most critical functions of the  CCB2004 are implemented in discrete logic. Among these functions are the following :

-    VME A24D16 slave interface;

- translation of all basic  signals from the TTCrq mezzanine to custom backplane;

- decoding of Hard_reset commands from Brcst[7..2] and Data[7..2] lines and generation of  500 ns Hard_Reset pulses to custom backplane;

- access to TTCrq board over I2C serial bus under VME control;

- JTAG access to FPGA/EPROM from VME ;

- monitoring of the Conf_Done signals from all connected peripheral or TF boards.

The FPGA can accept all the signals from the TTCrq board, and can generate all the backplane signals to peripheral boards, providing more flexibility than the discrete logic. The choice of interface between the TTCrq mezzanine and custom  backplane (discrete logic or FPGA) is defined by CSRA1[0] which is implemented in a discrete logic (Fig.2). When CSRA1[0]=1, the interface is a discrete logic interface (“discrete logic” mode). When CSRA1[0]=0, the interface is an FPGA (“FPGA” mode). In “FPGA” mode two options are possible: when TTC commands are coming from the TTCrq (if CSRB1[0]=0) and when they are generated  upon write into CSRB2 and CSRB3 (if CSRB1[0]=1).  Also, independently from CSRB1[0], several  commands (see Table 8, starting from Base+50 address) can be generated upon VME write to dedicated addresses. This option provides compatibility with the CCB’2001 board.

Figure 2:  Modes of Operation

After power cycling and  reloading from EPROM all the registers inside the FPGA are  set to “0”. This means all sources of L1ACC are enabled and the CCB’2004 will not hold the next L1ACC and Pretriggers after the first L1ACC has been sent to custom backplane.

2.  TTC  Interface and TTCrq Mezzanine Board

The list of signals that the CCB2004 accepts and transmits from/to the TTC mezzanine  board is  given in  Table 1.  Pin assignment of three mezzanine connectors is shown in Table 2. The board dimensions are given on Figure 3.

Table 1: Interface signals to/from TTC mezzanine card

| Signal | Bits | Short Description |
| --- | --- | --- |
| Signals to/from two 50-pin connectors backward compatible with TTCrm mezzanine card |  |  |
| BCntRes | 1 | Bunch Counter Reset signal |
| BCntStr | 1 | Bunch Counter Strobe. Indicates that a bunch number is present on the output BCnt<11:0> bus |
| Brcst<7:2> | 6 | Broadcast commands/data output bus |
| BrcstStr<2:1> | 2 | Broadcast messages strobes |
| Clock40Des1 | 1 | LHC 40.08 MHz deskewed reference clock signal |
| DbErStr | 1 | Indicates that a double error or a frame error has occurred |
| Dout<7:0> | 8 | Data bus. Normally used to output the data content of an individually-addressed commands/data |
| DQ<3:0> | 4 | Data qualifier bits. Indicate the type of data on the data bus register |
| BCnt<11..0> | 12 | Bunch or Event counter outputs |
| DoutStr | 1 | Data out strobe. Indicates valid data on the data bus |
| EvCntHStr | 1 | Event counter high word strobe |
| EvCntLStr | 1 | Event counter low word strobe |
| EvCntRes | 1 | Event counter reset signal |
| L1Accept | 1 | First level trigger accept signal |
| Reset_b | 1 | Reset TTCrx ASIC, active “low” |
| SinErrStr | 1 | Single error strobe |
| SubAddr<7:0> | 8 | Subaddress bus. Used to output the subaddress content of an individually address commands/data |
| TTCReady | 1 | Indicates that TTCrx ASIC is ready for normal operation |
| SDA | 1 | Data Line of I2C interface |
| SCL | 1 | Clock Line of I2C interface |
| Total | 54 |  |
| Signals to/from an additional 26-pin connector (TTCrq mezzanine only) |  |  |
| FoSelect<3..0> | 4 | Control inputs for the VCXO free running oscillation frequency (QPLL only) |
| Mode | 1 | QPLL multiplication mode control input |
| Restart | 1 | Enable/disable automatic restart of the PLL (control input for the QPLL) |
| ExternalControl | 1 | Control input for the VCXO (QPLL only) |
| Locked | 1 | QPLL output status signal |
| 40Mhz clock | 2 | LVDS clock outputs |
| 80/60Mhz clock | 2 | LVDS clock outputs |
| 160/120Mhz clock | 2 | LVDS clock outputs |
| 40Mhz_CMOS | 1 | CMOS 40Mhz clock output |
| ~Reset | 1 | QPLL control input, active “low” |
| Error | 1 | QPLL status output |
| Total | 17 |  |

The QPLL ASIC  residing on the TTCrq mezzanine card has several control inputs that are connected to the FPGA.  All control inputs are set into default states using on-board pull-up and pull-down resistors.

Table 2: Pin assignment of the TTCmr/TTCrq  mezzanine connectors

| Connector J1 |  |  | Connector J2 |  |  | Connector J3 (TTCrq only) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pin | Signal | Type | Pin | Signal | Type | Pin | Signal | Type |
| 1 | Clock40 | Output | 1 | BrcstStr2 | Output | 1 | F0Select<0> | Input |
| 2 | Clock40Des1 | Output | 2 | ClockL1Accept | Output | 2 | Mode | Input |
| 3 | Brcst<5> | Output | 3 | Brcst<6> | Output | 3 | InLVDS+ | Input |
| 4 | Brcst<4> | Output | 4 | Brcst<7> | Output | 4 | InLVDS- | Input |
| 5 | Brcst<3> | Output | 5 | EvCntRes | Output | 5 | GND | Power |
| 6 | Brcst<2> | Output | 6 | L1Accept | Output | 6 | ExternalClock | Input |
| 7 | Clock40Des2 | Output | 7 | EvCntLStr | Output | 7 | AutoRestart | Input |
| 8 | BrcstStr1 | Output | 8 | EvCntHStr | Output | 8 | ExternalContr | Input |
| 9 | DbErrStr | Output | 9 | BcntRes | Output | 9 | F0Select<3> | Input |
| 10 | SinErrStr | Output | 10 | GND | Power | 10 | ~Reset | Input |
| 11 | SubAdd<0> | Bidir | 11 | BCnt<0> | Output | 11 | Locked | Output |
| 12 | SubAdd<1> | Bidir | 12 | BCnt<1> | Output | 12 | Error | Output |
| 13 | SubAdd<2> | Bidir | 13 | BCnt<2> | Output | 13 | GND | Power |
| 14 | SubAdd<3> | Bidir | 14 | BCnt<3> | Output | 14 | Lvds80Mhz- | Output |
| 15 | SubAdd<4> | Bidir | 15 | BCnt<4> | Output | 15 | Lvds80Mhz+ | Output |
| 16 | SubAdd<5> | Bidir | 16 | BCnt<5> | Output | 16 | GND | Power |
| 17 | SubAdd<6> | Bidir | 17 | BCnt<6> | Output | 17 | F0Select<2> | Input |
| 18 | SubAdd<7> | Bidir | 18 | BCnt<7> | Output | 18 | GND | Power |
| 19 | DQ<0> | Output | 19 | BCnt<8> | Output | 19 | Lvds160Mhz+ | Output |
| 20 | DQ<1> | Output | 20 | BCnt<9> | Output | 20 | Lvds160Mhz- | Output |
| 21 | DQ<2> | Output | 21 | BCnt<10> | Output | 21 | GND | Power |
| 22 | DQ<3> | Output | 22 | BCnt<11> | Output | 22 | Lvds40Mhz- | Output |
| 23 | DoutStr | Output | 23 | JTAGTMS | Input | 23 | Lvds40Mhz+ | Output |
| 24 | GND | Power | 24 | JTAGTRST_b | Input | 24 | F0Select<1> | Input |
| 25 | Dout<0> | Bidir | 25 | JTAGTCK | Input | 25 | Cmos40Mhz | Output |
| 26 | Dout<1> | Bidir | 26 | JTAGTDO | Output | 26 | GND | Power |
| 27 | Dout<2> | Bidir | 27 | SDA | Bidir |  |  |  |
| 28 | Dout<3> | Bidir | 28 | JTAGTDI | Input |  |  |  |
| 29 | Dout<4> | Bidir | 29 | BCntStr | Output |  |  |  |
| 30 | Dout<5> | Bidir | 30 | Serial_B_Chan | Output |  |  |  |
| 31 | Dout<6> | Bidir | 31 | GND | Power |  |  |  |
| 32 | Dout<7> | Bidir | 32 | GND | Power |  |  |  |
| 33 | Reset_b | Input | 33 | GND | Power |  |  |  |
| 34 | TTCReady | Output | 34 | GND | Power |  |  |  |
| 35 | GND | Power | 35 | +5V | Power |  |  |  |
| 36 | GND | Power | 36 | +5V | Power |  |  |  |
| 37 | GND | Power | 37 | +5V | Power |  |  |  |
| 38 | GND | Power | 38 | +5V | Power |  |  |  |
| 39 | GND | Power | 39 | QPLL power | Passive |  |  |  |
| 40 | GND | Power | 40 | SCL | Input |  |  |  |
| 41 | GND | Power | 41 | GND | Power |  |  |  |
| 42 | GND | Power | 42 | GND | Power |  |  |  |
| 43 | GND | Power | 43 | +5V | Power |  |  |  |
| 44 | GND | Power | 44 | +5V | Power |  |  |  |
| 45 | GND | Power | 45 | +5V | Power |  |  |  |
| 46 | GND | Power | 46 | +5V | Power |  |  |  |
| 47 | GND | Power | 47 | GND | Power |  |  |  |
| 48 | GND | Power | 48 | GND | Power |  |  |  |
| 49 | GND | Power | 49 | GND | Power |  |  |  |
| 50 | GND | Power | 50 | GND | Power |  |  |  |

Figure 3:  TTCrq layout and dimensions

Each TTCrq mezzanine board has a unique 14-bit number that is defined by on-board jumpers. For more details see chapter 8 of the TTCrx Reference Manual [3]. This number can be accessed via I2C bus (see section 6) or from CSRB18 (see section 4.21).  The settings on TTCrq mezzanines installed on production CCB2004 boards are the following: SubAdd[7..0]=1 and Data[7..0]=1..255. As a rule, the code on Data[7..0] lines is equal to serial number of the CCB2004 board (marked on the front panel and on the top side of the main board). The other jumpers on TTCrq mezzanine described in [5] are:

- R53..R55 (termination resistors for 40/80/160Mhz LVDS clock outputs) are not installed  (there are 100 Ohm  termination resistors R15..R17 installed on CCB2004)

- ST19 is set to “TTCrx” clock source for clock pin 2 on J1 connector

- R59 is not mounted (on-board regulator provides the QPLL power)

- Custom  Backplane Interface

The lists of signals that the CCB2004 distributes to the custom peripheral and Track Finder backplanes are  given in Tables 3A and 3B respectively. Pin assignment of the  custom peripheral backplane connectors is based on [6] (both on-board connectors are  AMP 100145-1 female 125-pin  25-row by 5 pins) and given in Tables 4A and 5A. The CCB2004 may also be used in the CSC Track Finder crate with another custom backplane [7]. In this case the CCB2004 distributes/receives a subset of signals described in Tables 4B and 5B.

Texas Instruments SN74GTLPH16912GR transceivers  are  used for the communication with other modules in a crate. All GTLP backplane signals are sent in “negative” logic as proposed in [1], i.e. active output signal corresponds to a “low”  GTLP level. The same is expected for all incoming backplane signals. 28 point-to-point cfg_done signals (from TMB, DMB and MPC) are terminated on CCB2004 board  with 56 Ohm resistors to +1.5V. All bussed signals are terminated on both ends of  the backplane.

Table 3A: Peripheral Backplane Interface Signals

| Signal | Bits | Source | Destination | Type | Logic | Duration/Level |
| --- | --- | --- | --- | --- | --- | --- |
| Clock Bus: Clock Distribution & Bunch Crossing |  |  |  |  |  |  |
| ccb_clock40 | 19 | TTCrx, QPLL, osc | All 19 Slots | Point-to-point | LVDS | 40.08MHz |
| Clk80_MPC | 1 | QPLL, oscillator | MPC | Point-to-point | LVDS | 80.16MHz |
| Fast Control Bus |  |  |  |  |  |  |
| ccb_clock40_enable | 1 | TTCrx, QPLL, oscillator | All 19 Slots | Bussed | GTLP | 40.08Mhz |
| ccb_cmd[5..0] | 6 | TTCrx | All 19 Slots | Bussed | GTLP | Level |
| Ccb_ttcrx_ready (former ccb_reserved[0]) | 1 | TTCrx | All 19 slots | Bussed | GTLP | Level |
| QPLL_locked (former ccb_reserved[1] | 1 | QPLL ASIC | All 19 Slots | Bussed | GTLP | Level |
| ccb_evcntres | 1 | TTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| ccb_bcntres | 1 | TTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| Ccb_L1Reset (former ccb_reserved[4]) | 1 | DTTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| ccb_cmd_strobe | 1 | TTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| ccb_bx0 | 1 | DTTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| ccb_l1accept | 1 | TTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| ccb_data[7..0] | 8 | TTCrx | All 19 Slots | Bussed | GTLP | Level |
| ccb_data_strobe | 1 | TTCrx | All 19 Slots | Bussed | GTLP | 25 ns |
| ccb_reserved[3..2] | 2 | VME | All 19 Slots | Bussed | GTLP | Level |
| Total | 26 |  |  |  |  |  |
| TMB Reload Bus: ALCT+CLCT+TMB FPGA Reload |  |  |  |  |  |  |
| tmb_hard_reset | 1 | DTTCrx (*) | 9 TMB | Bussed | GTLP | 400 ns |
| tmb_cfg_done[8..0] | 9 | 9 TMB | VME | Point-to-Point | GTLP | Level |
| alct_hard_reset | 1 | DTTCrx (*) | 9 TMB | Bussed | GTLP | 400 ns |
| alct_cfg_done[8..0] | 9 | 9 TMB (from ALCT) | VME | Point-to-Point | GTLP | Level |
| Tmb_soft_reset (former tmb_reserved[1]) | 1 | VME | 9 TMB | Bussed | GTLP | Level |
| tmb_reserved[0] | 1 | VME | 9 TMB | Bussed | GTLP | Level |
| Total | 22 |  |  |  |  |  |
| MPC Reload Bus: MPC FPGA Reload |  |  |  |  |  |  |
| mpc_hard_reset | 1 | DTTCrx (*) | MPC | Point-to-Point | GTLP | 400 ns |
| mpc_cfg_done | 1 | MPC | CCB | Point-to-Point | GTLP | Level |
| Mpc_soft_reset (former mpc_reserved[2]) | 1 | VME | MPC | Point-to-Point | GTLP | Level |
| mpc_reserved[1..0] | 2 | VME | MPC | Point-to-Point | GTLP | Level |
| Total | 5 |  |  |  |  |  |
| DMB Reload Bus: DMB FPGA Reload |  |  |  |  |  |  |
| dmb_hard_reset | 1 | DTTCrx (*) | 9 DMB | Bussed | GTLP | 400 ns |
| dmb_cfg_done[8..0] | 9 | 9 DMB | CCB | Point-to-Point | GTLP | Level |
| Dmb_soft_reset (former dmb_reserved[2]) | 1 | VME | 9 DMB | Bussed | GTLP | Level |
| dmb_reserved[1..0] | 2 | VME | 9 DMB | Bussed | GTLP | Level |
| Total | 13 |  |  |  |  |  |
| DAQ Special Purpose Bus [Used by DMB and TMB] |  |  |  |  |  |  |
| dmb_cfeb_calibrate[2..0] | 3 | VME | 9 DMB 9TMB | Bussed | GTLP | Level |
| dmb_l1a_release | 1 | 9 DMB, 9 TMB | VME | Bussed | GTLP | Level |
| dmb_reserved_out[4..0] | 5 | VME | 9 DMB 9TMB | Bussed | GTLP | Level |
| dmb_reserved_in[2..0] | 3 | 9 DMB, 9 TMB | VME | Bussed | GTLP | Level |
| Total | 12 |  |  |  |  |  |
| Trigger Special Purpose Bus [Used by TMB only] |  |  |  |  |  |  |
| alct_adb_pulse_sync | 1 | VME | 9 TMB | Bussed | GTLP | Level |
| alct_adb_pulse_async | 1 | VME | 9 TMB | Bussed | GTLP | Level |
| clct_external_trigger | 1 | VME | 9 TMB | Bussed | GTLP | Level |
| alct_external_trigger | 1 | VME | 9 TMB | Bussed | GTLP | Level |
| clct_status[8..0] | 9 | 9 TMB | FP | Bussed | GTLP | Level |
| alct_status[8..0] | 9 | 9 TMB | FP | Bussed | GTLP | Level |
| tmb_l1a_request | 1 | 9 TMB | VME | Bussed | GTLP | Level |
| tmb_l1a_release | 1 | 9 TMB | VME | Bussed | GTLP | Level |
| tmb_reserved_in[4..0] | 5 | 9 TMB | VME | Bussed | GTLP | Level |
| tmb_reserved_out[2..0] | 3 | VME | 9 TMB | Bussed | GTLP | Level |
| Total | 32 |  |  |  |  |  |

(*) – decoded from Brcst[7..2] or Data[7..2]

Table 3B: Track Finder  Backplane Interface Signals

| Signal | Bits | Source | Destination | Type | Logic | Duration/Level |
| --- | --- | --- | --- | --- | --- | --- |
| Clock Bus: Clock Distribution & Bunch Crossing |  |  |  |  |  |  |
| ccb_clock40 | 17 | TTCrx, QPLL, oscillator | All 17 Slots | Point-to-point | LVDS | 40.08MHz or 80.16Mhz, see section 2.1 |
| Clk80_MPC | 3 | QPLL, oscillator | 3 MPC | Point-to-point | LVDS | 80.16MHz |
| Fast Control Bus |  |  |  |  |  |  |
| ccb_clock40_enable | 1 | TTCrx, QPLL, oscillator | All 17 Slots | Bussed | GTLP | 40.08Mhz |
| ccb_cmd[5..0] | 6 | TTCrx | All 17 Slots | Bussed | GTLP | Level |
| Ccb_ttcrx_ready (former ccb_reserved[0]) | 1 | TTCrx | All 17 slots | Bussed | GTLP | Level |
| QPLL_locked (former ccb_reserved[1] | 1 | QPLL ASIC | All 17 Slots | Bussed | GTLP | Level |
| ccb_evcntres | 1 | TTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| ccb_bcntres | 1 | TTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| Ccb_L1Reset (former ccb_reserved[4]) | 1 | DTTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| ccb_cmd_strobe | 1 | TTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| ccb_bx0 | 1 | DTTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| ccb_l1accept | 1 | TTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| ccb_data[7..0] | 8 | TTCrx | All 17 Slots | Bussed | GTLP | Level |
| ccb_data_strobe | 1 | TTCrx | All 17 Slots | Bussed | GTLP | 25 ns |
| ccb_reserved[3..2] | 2 | VME | All 17 Slots | Bussed | GTLP | Level |
| Total | 26 |  |  |  |  |  |
| SP, MS, MPC, DDU Reload Bus |  |  |  |  |  |  |
| sp_hard_reset | 1 | DTTCrx (*) | 12 SP | Bussed | GTLP | 400 ns |
| sp_cfg_done[12..1] | 12 | 12 SP | VME | Point-to-Point | GTLP | Level |
| ms_hard_reset | 1 | DTTCrx (*) | MS | Point-to-Point | GTLP | 400 ns |
| ms_cfg_done | 1 | MS | VME | Point-to-Point | GTLP | Level |
| mpc_hard_reset | 1 | DTTCtx(*) | 3 MPC | Bussed | GTLP | 400 ns |
| mpc_cfg_done | 3 | 3 MPC | VME | Point-to-Point | GTLP | Level |
| ddu_cfg_done | 1 | DDU | VME | Point-to-Point | GTLP | Level |
| Total | 20 |  |  |  |  |  |
| Special Purpose and Reserved Lines |  |  |  |  |  |  |
| sp_l1a_request | 1 | 12 SP | VME | Bussed | GTLP | Pulse |
| ms_l1a_request | 1 | MS | VME | Point-to-Point | GTLP | Pulse |
| ms_to_ccb[1..0] | 2 | MS | VME | Point-to-Point | GTLP |  |
| sp_to-ccb[2..0] | 3 | 12 SP | VME | Bussed | GTLP |  |
| ccb_to_ms[3..0] | 4 | CCB | MS | Point-to-Point | GTLP |  |
| Total | 11 |  |  |  |  |  |

(*) – decoded from Brcst[7..2] or Data[7..2]

## Table 4A: X40 Backplane Connector, Peripheral Crate

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | CLK+DMB1 | B1 | CLK-DMB1 | C1 | GND | D1 | CLK+TMB1 | E1 | CLK-TMB1 |
| A2 | CLK+TMB2 | B2 | CLK-TMB2 | C2 | GND | D2 | CLK+DMB9 | E2 | CLK-DMB9 |
| A3 | CLK+DMB2 | B3 | CLK-DMB2 | C3 | GND | D3 | CLK+TMB9 | E3 | CLK-TMB9 |
| A4 | CLK+TMB3 | B4 | CLK-TMB3 | C4 | GND | D4 | CLK+DMB8 | E4 | CLK-DMB8 |
| A5 | CLK+DMB3 | B5 | CLK-DMB3 | C5 | GND | D5 | CLK+TMB8 | E5 | CLK-TMB8 |
| A6 | CLK+TMB4 | B6 | CLK-TMB4 | C6 | GND | D6 | CLK+DMB7 | E6 | CLK-DMB7 |
| A7 | CLK+DMB4 | B7 | CLK-DMB4 | C7 | GND | D7 | CLK+TMB7 | E7 | CLK-TMB7 |
| A8 | CLK+TMB5 | B8 | CLK-TMB5 | C8 | GND | D8 | CLK+DMB6 | E8 | CLK-DMB6 |
| A9 | CLK+DMB5 | B9 | CLK-DMB5 | C9 | GND | D9 | CLK+TMB6 | E9 | CLK-TMB6 |
| A10 | CLK+MPC | B10 | CLK-MPC | C10 | GND | D10 | alct_cfg_d_9 | E10 | dmb_cfg_d_9 |
| A11 | tmb_cfg_d_1 | B11 | alct_cfg_d_1 | C11 | GND | D11 | dmb_cfg_d_8 | E11 | tmb_cfg_d_9 |
| A12 | dmb_cfg_d_1 | B12 | tmb_cfg_d_2 | C12 | GND | D12 | tmb_cfg_d_8 | E12 | alct_cfg_d_8 |
| A13 | alct_cfg_d_2 | B13 | dmb_cfg_d_2 | C13 | GND | D13 | alct_cfg_d_7 | E13 | dmb_cfg_d_7 |
| A14 | tmb_cfg_d_3 | B14 | alct_cfg_d_3 | C14 | GND | D14 | dmb_cfg_d_6 | E14 | tmb_cfg_d_7 |
| A15 | dmb_cfg_d_3 | B15 | tmb_cfg_d_4 | C15 | GND | D15 | tmb_cfg_d_6 | E15 | alct_cfg_d_6 |
| A16 | alct_cfg_d_4 | B16 | dmb_cfg_d_4 | C16 | GND | D16 | Mpc_h_reset | E16 | Mpc_rsv0 |
| A17 | Tmb_cfg_d_5 | B17 | alct_cfg_d_5 | C17 | GND | D17 | Mpc_rsv1 | E17 | Mpc_softres |
| A18 | dmb_cfg_d_5 | B18 |  | C18 | GND | D18 | mpc_c_done | E18 |  |
| A19 | Clock_enable | B19 | Ccb_rsv4 | C19 | GND | D19 | Clk80+MPC | E19 | Clk80-MPC |
| A20 | Ccb_cmd0 | B20 | Ccb_cmd1 | C20 | GND | D20 | Ccb_cmd2 | E20 | Ccb_cmd3 |
| A21 | Ccb_cmd4 | B21 | Ccb_cmd5 | C21 | GND | D21 | Ccb_evcntres | E21 | Ccb_bcntres |
| A22 | Ccb_cmd_str | B22 | Ccb_bx0 | C22 | GND | D22 | Ccb_l1accept | E22 | Ccb_data_st |
| A23 | Ccb_data0 | B23 | Ccb_data1 | C23 | GND | D23 | Ccb_data2 | E23 | Ccb_data3 |
| A24 | Ccb_data4 | B24 | Ccb_data5 | C24 | GND | D24 | Ccb_data6 | E24 | Ccb_data7 |
| A25 | Ccb_rsv0 | B25 | Ccb_rsv1 | C25 | GND | D25 | Ccb_rsv2 | E25 | Ccb_rsv3 |

## Table 4B: X40 Backplane Connector, Track Finder Crate

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | CLK+DDU | B1 | CLK-DDU | C1 | GND | D1 |  | E1 |  |
| A2 | CLK+MPC1 | B2 | CLK-MPC1 | C2 | GND | D2 |  | E2 |  |
| A3 | CLK+MPC2 | B3 | CLK-MPC2 | C3 | GND | D3 | CLK+SP12 | E3 | CLK-SP12 |
| A4 | CLK+MPC3 | B4 | CLK-MPC3 | C4 | GND | D4 | CLK+SP11 | E4 | CLK-SP11 |
| A5 | CLK+SP1 | B5 | CLK-SP1 | C5 | GND | D5 | CLK+SP10 | E5 | CLK-SP10 |
| A6 | CLK+SP2 | B6 | CLK-SP2 | C6 | GND | D6 | CLK+SP9 | E6 | CLK-SP9 |
| A7 | CLK+SP3 | B7 | CLK-SP3 | C7 | GND | D7 | CLK+SP8 | E7 | CLK-SP8 |
| A8 | CLK+SP4 | B8 | CLK-SP4 | C8 | GND | D8 | CLK+SP7 | E8 | CLK-SP7 |
| A9 | CLK+SP5 | B9 | CLK-SP5 | C9 | GND | D9 | CLK+MS | E9 | CLK-MS |
| A10 | CLK+SP6 | B10 | CLK-SP6 | C10 | GND | D10 |  | E10 | sp_c_don_12 |
| A11 |  | B11 |  | C11 | GND | D11 |  | E11 | sp_c_don_11 |
| A12 |  | B12 |  | C12 | GND | D12 |  | E12 | sp_c_don_10 |
| A13 | ddu_cfg_done | B13 | mpc_c_don_1 | C13 | GND | D13 |  | E13 | sp_c_done_9 |
| A14 | mpc_c_done_2 | B14 | mpc_c_don_3 | C14 | GND | D14 |  | E14 | sp_c_done_8 |
| A15 | sp_c_done_1 | B15 | sp_c_done_2 | C15 | GND | D15 |  | E15 | sp_c_done_7 |
| A16 | sp_c_done_3 | B16 | sp_c_done_4 | C16 | GND | D16 |  | E16 |  |
| A17 | sp_c_done_5 | B17 | sp_c_done_6 | C17 | GND | D17 |  | E17 |  |
| A18 |  | B18 |  | C18 | GND | D18 | ms_c_done | E18 |  |
| A19 | Clock_enable | B19 | Ccb_rsv4 | C19 | GND | D19 |  | E19 |  |
| A20 | Ccb_cmd0 | B20 | Ccb_cmd1 | C20 | GND | D20 | Ccb_cmd2 | E20 | Ccb_cmd3 |
| A21 | Ccb_cmd4 | B21 | Ccb_cmd5 | C21 | GND | D21 | Ccb_evcntres | E21 | Ccb_bcntres |
| A22 | Ccb_cmd_str | B22 | Ccb_bx0 | C22 | GND | D22 | Ccb_l1accept | E22 | Ccb_data_st |
| A23 | Ccb_data0 | B23 | Ccb_data1 | C23 | GND | D23 | Ccb_data2 | E23 | Ccb_data3 |
| A24 | Ccb_data4 | B24 | Ccb_data5 | C24 | GND | D24 | Ccb_data6 | E24 | Ccb_data7 |
| A25 | Ccb_rsv0 | B25 | Ccb_rsv1 | C25 | GND | D25 | Ccb_rsv2 | E25 | Ccb_rsv3 |

## Table 5A: X41 Backplane Connector, Peripheral Crate

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | Tmb_h_reset | B1 | Alct_h_reset | C1 | GND | D1 | tmb_rsv_0 | E1 | tmb_softres |
| A2 | Alct_adb_p_s | B2 | alct_adb_p_a | C2 | GND | D2 | clct_ext_trig | E2 | alct_ext_trig |
| A3 | Clct_status0 | B3 | Clct_status1 | C3 | ### GND | D3 | Clct_status2 | E3 | Clct_status3 |
| A4 | Clct_status4 | B4 | Clct_status5 | C4 | GND | D4 | Clct_status6 | E4 | Clct_status7 |
| A5 | Clct_status8 | B5 | Alct_status0 | C5 | GND | D5 | alct_status1 | E5 | alct_status2 |
| A6 | Alct_status3 | B6 | Alct_status4 | C6 | GND | D6 | Alct_status5 | E6 | Alct_status6 |
| A7 | Alct_status7 | B7 | alct_status8 | C7 | GND | D7 | tmb_l1a_req | E7 | tmb_l1a_rel |
| A8 | Tmb_rsv_in0 | B8 | Tmb_rsv_in1 | C8 | GND | D8 | Tmb_rsv_in2 | E8 | Tmb_rsv_in3 |
| A9 | Tmb_rsv_in4 | B9 | tmb_rsv_o0 | C9 | GND | D9 | Tmb_rsv_o1 | E9 | Tmb_rsv_o2 |
| A10 | dmb_h_reset | B10 | dmb_rsv_0 | C10 | GND | D10 | dmb_rsv_1 | E10 | dmb_softres |
| A11 | dmb_cfeb_c0 | B11 | dmb_cfeb_c1 | C11 | GND | D11 | dmb_cfeb_c2 | E11 | dmb_l1a_rel |
| A12 | dmb_rsv_o0 | B12 | dmb_rsv_o1 | C12 | GND | D12 | dmb_rsv_o2 | E12 | dmb_rsv_o3 |
| A13 | dmb_rsv_o4 | B13 | dmb_rsv_in0 | C13 | GND | D13 | dmb_rsv_in1 | E13 |  |
| A14 |  | B14 |  | C14 | +1.5V | D14 |  | E14 |  |
| A15 |  | B15 |  | C15 | GND | D15 |  | E15 |  |
| A16 |  | B16 |  | C16 | +1.5V | D16 |  | E16 |  |
| A17 |  | B17 |  | C17 | GND | D17 |  | E17 |  |
| A18 |  | B18 |  | C18 | +1.5V | D18 |  | E18 |  |
| A19 |  | B19 |  | C19 | GND | D19 |  | E19 |  |
| A20 |  | B20 |  | C20 | +1.5V | D20 |  | E20 |  |
| A21 |  | B21 |  | C21 | GND | D21 |  | E21 |  |
| A22 |  | B22 |  | C22 | +1.5V | D22 |  | E22 |  |
| A23 |  | B23 |  | C23 | GND | D23 |  | E23 |  |
| A24 |  | B24 |  | C24 | +1.5V | D24 |  | E24 |  |
| A25 |  | B25 |  | C25 | GND | D25 |  | E25 |  |

## Table 5B: X41 Backplane Connector, Track Finder Crate

| Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal | Pin | Signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | sp_hard_reset | B1 | ms_h_reset | C1 | GND | D1 |  | E1 |  |
| A2 |  | B2 |  | C2 | GND | D2 |  | E2 |  |
| A3 |  | B3 |  | C3 | ### GND | D3 |  | E3 |  |
| A4 |  | B4 |  | C4 | GND | D4 |  | E4 |  |
| A5 |  | B5 |  | C5 | GND | D5 |  | E5 |  |
| A6 |  | B6 |  | C6 | GND | D6 |  | E6 |  |
| A7 |  | B7 |  | C7 | GND | D7 | sp_l1a_req | E7 | ms_l1a_req |
| A8 | ms_to_ccb_0 | B8 | ms_to_ccb_1 | C8 | GND | D8 |  | E8 |  |
| A9 |  | B9 |  | C9 | GND | D9 |  | E9 |  |
| A10 | mpc_h_reset | B10 |  | C10 | GND | D10 |  | E10 |  |
| A11 |  | B11 |  | C11 | GND | D11 |  | E11 |  |
| A12 | ccb_to_ms_0 | B12 | ccb_to_ms_1 | C12 | GND | D12 | ccb_to_ms_2 | E12 | ccb_to_ms_3 |
| A13 |  | B13 | sp_to_ccb_0 | C13 | GND | D13 | sp_to_ccb_1 | E13 | sp_to_ccb_2 |
| A14 |  | B14 |  | C14 | +1.5V | D14 | Clk80+Mpc1 | E14 | Clk80-Mpc1 |
| A15 |  | B15 |  | C15 | GND | D15 | Clk80+Mpc2 | E15 | Clk80-Mpc2 |
| A16 |  | B16 |  | C16 | +1.5V | D16 | Clk80+Mpc3 | E16 | Clk80-Mpc3 |
| A17 |  | B17 |  | C17 | GND | D17 |  | E17 |  |
| A18 |  | B18 |  | C18 | +1.5V | D18 |  | E18 |  |
| A19 |  | B19 |  | C19 | GND | D19 |  | E19 |  |
| A20 |  | B20 |  | C20 | +1.5V | D20 |  | E20 |  |
| A21 |  | B21 |  | C21 | GND | D21 |  | E21 |  |
| A22 |  | B22 |  | C22 | +1.5V | D22 |  | E22 |  |
| A23 |  | B23 |  | C23 | GND | D23 |  | E23 |  |
| A24 |  | B24 |  | C24 | +1.5V | D24 |  | E24 |  |
| A25 |  | B25 |  | C25 | GND | D25 |  | E25 |  |

3.1.  Clock Distribution

There are four sources of the 40.078Mhz clock on a CCB2004 board:  Clock40Des1 from the TTCrx ASIC (1), CMOS40Mhz output from the QPLL ASIC (2), QPLL_40Mhz  after LVDS-to-LVTTL conversion (3) and Clock40Osc (4) that is derived from an on-board oscillator (80.157Mhz divided by 2). One source must be selected using switch S5.

There are two sources of the 80.157Mhz: Clock80Mhz from  the QPLL ASIC and Clock80Osc from an on-board oscillator. One source must be selected using switch S7.

All modules in the peripheral crate require a point-to-point 40MHz LVDS clock from the CCB2004. Some modules in the Track Finder crate may require either 40Mhz or 80Mhz clocks over the same lines from the CCB’2004. In order to allow the CCB2004 design to be compatible with both peripheral and Track Finder crates the CCB2004 has a switch that allows to chose 40/80Mhz source for a several slots (see Table 6).

Table 6: Clocking Options in the Peripheral and Track Finder Crates

| Slot | Peripheral Crate | Clock Source | Track Finder Crate | Clock Source |
| --- | --- | --- | --- | --- |
| 1 | Crate Master | - | Crate Master | - |
| 2 | TMB1 | 40Mhz | DDU | 40Mhz + 80Mhz |
| 3 | DMB1 | 40Mhz | MPC1 | 40Mhz + 80Mhz |
| 4 | TMB2 | 40Mhz | MPC2 | 40Mhz + 80Mhz |
| 5 | DMB2 | 40Mhz | MPC3 | 40Mhz |
| 6 | TMB3 | 40Mhz | SP1 | 40Mhz or 80Mhz |
| 7 | DMB3 | 40Mhz | SP2 | 40Mhz or 80Mhz |
| 8 | TMB4 | 40Mhz | SP3 | 40Mhz or 80Mhz |
| 9 | DMB4 | 40Mhz | SP4 | 40Mhz or 80Mhz |
| 10 | TMB5 | 40Mhz | SP5 | 40Mhz or 80Mhz |
| 11 | DMB5 | 40Mhz | SP6 | 40Mhz or 80Mhz |
| 12 | MPC | 40Mhz + 80Mhz | CCB | TTC/internal |
| 13 | CCB | internal | - | - |
| 14 | TMB6 | 40Mhz | Muon Sorter | 40Mhz |
| 15 | DMB6 | 40Mhz | - | - |
| 16 | TMB7 | 40Mhz | SP7 | 40Mhz or 80Mhz |
| 17 | DMB7 | 40Mhz | SP8 | 40Mhz or 80Mhz |
| 18 | TMB8 | 40Mhz | SP9 | 40Mhz or 80Mhz |
| 19 | DMB8 | 40Mhz | SP10 | 40Mhz or 80Mhz |
| 20 | TMB9 | 40Mhz | SP11 | 40Mhz or 80Mhz |
| 21 | DMB9 | 40Mhz | SP12 | 40Mhz or 80Mhz |

All clocks are distributed over the peripheral backplane using individual point-to-point LVDS lines.  There are fine and coarse delays for the 40Mhz clock and command signals incorporated into TTCrx ASIC. They are programmable via the I2C interface or optical path.  In addition to individual clock per each slot there is a common GTLP line (formerly assigned for ccb_clock_enable signal) that carries a continuous 40.078Mhz signal selected from one of four sources described above. This signal can be used by Sector Processors  in conjunction with the 80Mhz clock.

A dedicated 80Mhz LVDS clock in addition to 40Mhz clock is transmitted to MPC slot in the peripheral crate and to every (out of 3) MPC slot in the Track Finder crate.

  - .  Fast  Control  and Data Buses

The TTCrx ASIC can distribute broadcast commands over Brcst[7..2] bus and individually addressed commands over Data[7..0] and Subadd[7..0] buses. Six backplane lines ccb_cmd[5..0] and ccb_cmd_strobe lines represent the Brcst[7..2] and BrcstStr1 outputs of the TTCrx respectively. The ccb_data[7..0] and ccb_data_strobe lines represent the data[7..0] and data_strobe outputs of the TTCrx respectively. Peripheral EMU electronics may need to decode both broadcast (all crates in a system) and individual (only one crate in a system) commands.  The same decoding scheme (Table 7) is being used on both fast control and data buses.  Subadd[7..0] lines are not provided the peripheral backplane. Note that after power up several outputs of TTCrx, including Dout[7..0], DQ[3..0], SubAdd[7..0] and DoutStr are disabled (chapter 3 of [3]). In order to enable them, bit 5 in the control register of the TTCrx should be set “1”.

According to [3], both Clk40Des1 and Clk40Des2 can be used for the synchonization of BrcstStr2 and Brcst[7..6]. The BrcstStr1 and Brcst[5..2] are always synchronized with the Clk40des1.  Also there are separate settings for coarse delays for BrcsStr1 and BrcstStr2. So, in order to make a broadcast scheme work in the CCB’2004, the coarse settings for BrcstStr1 and BrcstStr2 should be the same and BrcsStr2 and Brcst[7..6] should be synchronized with the Clk40Des1. This can be done by control access to the TTCrx ASIC. This is a default state after power up and reset.

Several Hard_Reset commands (from either Brcst[7..2] or Data[7..2] busses) are decoded inside the CCB2004 (they are marked red in Table 7).  All other  commands must be decoded by targeted peripheral boards from the ccb_cmd[5..0]+ccb_cmd_strobe  AND ccb_data[7..2]+ccb_data_strobe  lines.

Table 7: Fast Control Bus Ccb_cmd[5..0] and Ccb_data[7..2] (*) Commands

| Signal | Code (hex) | Description |
| --- | --- | --- |
| BC0 | 01 | Bunch Crossing Zero |
| OC0 | 02 | Orbit Counter Reset |
| L1 Reset | 03 | Reset L1 readout buffers |
| Hard_reset (**) | 04 | Reload all FPGAs from EPROMs |
| Start Trigger | 06 |  |
| Stop Trigger | 07 |  |
| Test Enable | 08 |  |
| Private Gap | 09 |  |
| Private Orbit | 0A |  |
| CCB_hard_reset (***) | 0F | Reload Xilinx FPGA from its EPROM on CCB’2004 board |
| Tmb_hard_reset (**) | 10 | Reload TMB FPGA’s from EPROM |
| Alct_hard_reset (**) | 11 | Reload ALCT FPGA’s from EPROM |
| Dmb_hard_reset (**) | 12 | Reload DMB FPGA’s from EPROM |
| Mpc_hard_reset (**) | 13 | Reload MPC FPGA’s from EPROM |
| Dmb_cfeb_calibrate0 | 14 | CFEB Calibrate Pre-Amp Gain |
| Dmb_cfeb_calibrate1 | 15 | CFEB Trigger Pattern Calibration |
| Dmb_cfeb_calibrate2 | 16 | CFEB Pedestal Calibration |
| Dmb_cfeb_initiate | 17 | Initiate CFEB calibration (Hold next L1ACC and Pretriggers) |
| Alct_adb_pulse_sync | 18 | Pulse Anode Discriminator, synchronous |
| Alct_adb_pulse_async | 19 | Pulse Anode Discriminator, asynchronous |
| Clct_external_trigger | 1A | External Trigger All CLCTs |
| Alct_external_trigger | 1B | External Trigger All ALCTs |
| Soft_reset | 1C | Initializes the FPGA on DMB, TMB and MPC boards |
| DMB_soft_reset | 1D | Initializes the FPGA on a DMB |
| TMB_soft_reset | 1E | Initializes the FPGA on a TMB |
| MPC_soft_reset | 1F | Initializes the FPGA on a MPC |
| Inject patterns from TMBs | 24 | Inject test patterns from the TMB’s internal RAM to MPC |
| Alct_adb_pulse | 25 | Generate both synchronous and asynchronous anode discriminator pulses |
| Inject patterns from SP | 2F | Inject test patterns from the SP’s internal FIFO to MS |
| Inject patterns from MPC | 30 | Inject test patterns from the MPC’s internal FIFO to SP |
| Inject patterns from MS | 31 | Inject test patterns from the MS’s internal FIFO to GMT |
| Bunch Counter Reset | 32 | Resets bunch counters |

(*) – data[1..0] can be any values

(**) – decoded by CCB

(***) –decoded only by discrete logic interface of the CCB from the TTCrq outputs

  - .  L1ACC  and  Pretrigger  Sources  and Control

There are ten possible sources for  L1ACC and two Pretriggers (ALCT_external_trigger and CLCT_external_trigger) in “FPGA” mode:

- TTCrx_L1ACC (25 ns pulse),

- External_L1ACC (LVDS input coming through the front panel),

- Tmb_l1a_request (25 ns pulse coming  over custom backplane),

- Tmb_l1a_release (25 ns pulse coming over custom backplane)

- Artificial L1ACC generated upon VME write to (base + 54) address (see Table 8),

- Any source of ALCT_adb_pulse_sync (VME,  TTC command),

- Any source of ALCT_adb_pulse_async (VME, TTC command)

- DMB_cfeb_calibrate[2..0] pulses (VME, TTC command);  logical “OR” of three pulses.

Each of these sources can be masked  using dedicated CSRB1 bits  (disabled if “1” and enabled if “0”). Before distribution to backplane both Pretriggers can be delayed for a number of 40Mhz clocks (from 1 to 255, defined by CSRB5[15..8]). Independently from Pretriggers,  the L1ACC can also be delayed for n=1..255 clock cycles where (n) is defined by CSRB5[7..0]. The ALCT_adb_pulse_sync and ALCT_adb_pulse_async backplane signals are sent to backplane without delay.  These sources may produce L1ACC to backplane when CSRA1[0]=0 independently from CSRB1[0]. In “discrete logic” mode, when the CSRA1[0]=1, only L1ACC from TTCrx may  unconditionally propagate to custom backplane with a delay of ~50 ns.

There is a 32-bit counter of L1ACC requests. The counter counts the  L1ACC requests from any selected source in the FPGA even if the  transmission to backplane is disabled. The counter is disabled after power up and the CCB2004 internal reset.

In a special CFEB calibration mode, the CCB2004 may receive a dmb_cfeb_initiate command from the TTCrx (this command can be also send upon write to a specific VME address, see Table 8). In this mode, the CCB2004 “holds” the next L1ACC and both Pretriggers  (by preventing them from being broadcast on the backplane). In a similar way, if the CSRB1[13]=1, then the CCB2004 holds the next L1ACC and Pretriggers after the first L1ACC has been sent to backplane.  The CCB’2004 can exit from  this “hold” mode  when a dmb_l1a_release or tmb_l1a_release signals have been received from  DMB or TMB or when VME write to (base address + 58) has been performed.

  - Reloading  Signals

The Xilinx FPGA residing on CCB2004 board can be reloaded from its EPROM on common and CCB-specific “Hard_reset” commands (Table 7) if switch S10-8 is “on” (see Section 7). Reconfiguration time is ~60 ms.  Note that when common “Hard_Reset” or “CCB_Hard_reset” commands are decoded inside the FPGA, they in fact do not force  reconfiguration of the FPGA itself. These commands are effective for FPGA reconfiguration  only after decoding by discrete logic.

Hard_reset reloading signals (from either Brcst[7..2] or Data[7..2] busses)  are decoded by the CCB2004 and expanded to 500 ns before distribution to the backplane. There is also a common Hard_reset command (Table 7), that causes generation of all reloading signals to all boards in a crate simultaneously.

An ALCT, TMB, DMB and MPC cards inform the CCB2004 when they have  completed the reconfiguration process by asserting their cfg_done signals. These static signals are available for reading from CSRA2-3 (see 3.2-3.3).

- VME Interface and Control and Status Registers (CSR)

The CCB2004 can be accessed  in the VME crate using geographical addressing  that utilizes the address pins GA<4-0> available on the VME64x backplane. In this mode the CCB2004 recognizes  its address space when the code on address lines A<23-19> is equal to the 5-bit geographical code of its slot. The base address is 600000(hex) in the Track Finder crate (slot 12) and 680000(hex) in the peripheral crate (slot 13). The board  recognizes an AM codes 39(hex), 3D(hex) and supports A24D16 slave operations. The CCB2004 does not respond to byte-addressing modes, so all valid addresses must be even numbers.  The list of available VME addresses is given in Table 8.

There are two groups of Control and Status Registers (CSR): CSRA and CSRB. The group A is implemented in a discrete logic and the group B in Xilinx FPGA.

Table 8

| Address (hex) | Function |
| --- | --- |
| Group A, discrete logic |  |
| Base + 0 | CSRA1 (control/status register), discrete logic |
| Base + 2 | CSRA2 (status register), discrete logic. Write to CSRA2 causes generation of Hard_reset for the FPGA. See 4.2. for more details. |
| Base + 4 | CSRA3 (status register), discrete logic. Write to CSRA3 causes generation of Soft_reset for the FPGA See 4.3. for more details. |
| Group B, FPGA |  |
| Base + 20 | CSRB1 (control register), FPGA |
| Base + 22 | CSRB2 (command bus), FPGA |
| Base + 24 | CSRB3 (data bus), FPGA |
| Base + 26 | CSRB4 (general purpose R/W register, future use), FPGA |
| Base + 28 | CSRB5 (delay register), FPGA |
| Base + 2a | CSRB6 (control register), FPGA |
| Base + 2c | CSRB7 (QPLL control register), FPGA |
| Base + 2e | CSRB8 (general purpose R/W register, future use), FPGA |
| Base + 30 | CSRB9 (status register, serial ID chip), FPGA |
| Base + 32 | CSRB10, not implemented, FPGA |
| Base + 34 | CSRB11 (status register), FPGA |
| Base + 36 | CSRB12 (status register), FPGA |
| Base + 38 | CSRB13 (status register), FPGA |
| Base + 3a | CSRB14 (status register), FPGA |
| Base + 3c | CSRB15 (status register), FPGA |
| Base + 3e | CSRB16 (status register), FPGA |
| Base + 40 | CSRB17 (status register, date of firmware revision), FPGA |
| Base + 42 | CSRB18 (TTCrx hardwired ID number). Read only. Valid after TTCrx reset. CSRB18[7..0] = Data[7..0] and CSRB18[15..8] = SubAdr[7..0] |
| Base + 44 | CSRB19 (32-bit counter of BRCSTSTR1 strobe from the TTCrx; 16 LSB (Least Significant Bits); write any data to this address resets the 32-bit counter) |
| Base + 46 | CSRB19 (32-bit counter of BRCSTSTR1 strobe from the TTCrx; 16 MSB (Most Significant Bits) |
| Base + 48 | CSRB21 (16-bit counter of DOUTSTR strobes from the TTCrx; write any data to this address resets the counter) |
| Base + 4a | CSRB22 (16-bit counter of QPLL_LOCK signals from the QPLL; write any data to this address resets the counter) |
| Base + 4c | CSRB23 (16-bit counter of TTC_READY signals from the TTCrx; write any data to this address resets the counter) |
| Base + 4e | CSRB24 (16-bit counter of QPLL_ERR signals from the QPLL; write any data to this address resets the counter) |
| Base + 50 | Generate L1Reset 25 ns pulse (write only), FPGA (*) |
| Base + 52 | Generate BC0 25 ns pulse (write only), FPGA (*) |
| Base + 54 | Generate L1ACC 25 ns pulse (write only) and external triggers (if enabled), FPGA (*) |
| Base + 56 | Generate “DMB_cfeb_initiate” pulse (write only) to hold the next L1ACC and Pretriggers, FPGA |
| Base + 58 | Generate “Release HOLD L1ACC Mode” pulse to enable transmission of the L1ACC and Pretriggers (write only), FPGA |
| Base + 5a | Reset CSRB11 Error Bits (write only), FPGA |
| Base + 5c | Reset TTCrx ASIC (write only). Generates 50 us pulse of negative polarity to TTCrx. |
| Base + 5e |  |
| Base + 60 | Generate “Hard_Reset” 500 ns pulse to all modules in the crate (write only), FPGA (*) |
| Base + 62 | Generate “TMB_Hard_Reset” 500 ns pulse to all TMB boards in the crate (write only), FPGA (*) |
| Base + 64 | Generate “DMB_Hard_Reset” 500 ns pulse to all DMB boards in the crate (write only), FPGA (*) |
| Base + 66 | Generate “ALCT_Hard_Reset” 500 ns pulse to all ALCT boards (write only), FPGA (*) |
| Base + 68 | Generate “MPC_Hard_Reset” 500 ns pulse to MPC (write only), FPGA (*) |
| Base + 6a | Generate “Soft_Reset” 25 ns pulse to TMB, DMB, MPC boards in the crate (write only), FPGA (*) |
| Base + 6c | Generate “TMB_Soft_Reset” 25 ns pulse to TMB boards (write only), FPGA (*) |
| Base + 6e | Generate “DMB_Soft_Reset” 25 ns pulse to TMB boards (write only), FPGA (*) |
| Base + 70 | Generate “MPC_Soft_Reset” 25 ns pulse to TMB boards (write only), FPGA (*) |
| Base + 72 |  |
| Base + 74 |  |
| Base + 76 |  |
| Base + 78 |  |
| Base + 7a |  |
| Base + 7c |  |
| Base + 7e |  |
| Base + 80 | Generate both “ALCT_adb_pulse_sync” and “ALCT_adb_pulse_async” (write only), FPGA (*) |
| Base + 82 | Generate “ALCT_adb_pulse_sync” 500 ns pulse (write only), FPGA (*) |
| Base + 84 | Generate “ALCT_adb_pulse_async” pulse (write only), FPGA (*) |
| Base + 86 | Generate “CLCT_external_trigger” 25 ns pulse (write only), FPGA (*) |
| Base + 88 | Generate “ALCT_external_trigger” 25 ns pulse (write only), FPGA (*) |
| Base + 8a | Generate “DMB_cfeb_calibrate[0]” 25 ns pulse (write only), FPGA (*) |
| Base + 8c | Generate “DMB_cfeb_calibrate[1]” 25 ns pulse (write only), FPGA (*) |
| Base + 8e | Generate “DMB_cfeb_calibrate[2]” 25 ns pulse (write only), FPGA (*) |
| Base + 90 | Read L1ACC Counter[15..0] (read only), FPGA |
| Base + 92 | Read L1ACC Counter[31..16] (read only), FPGA |
| Base + 94 | Reset L1ACC Counter to 0 (write only), FPGA |
| Base + 96 | Enable L1ACC Counter to count (write only), FPGA |
| Base + 98 | Disable L1ACC Counter to count (write only), FPGA |
| Base + 9a | Generate 800 us “Reset pulse” on 1-Wire bus to initialize the serial ID chip (write only), FPGA |
| Base + 9c | Generate 3 us “Read pulse” on 1-Wire bus to read data from the serial ID chip (write only), FPGA |
| Base + 9e | Reset CSRB9 (write only), FPGA |
| Base + a0 | Generate “Write-zero” 50 us pulse on 1-Wire bus to send a command to serial ID chip (write only), FPGA |
| Base + a2 | Generate “Write-one” 12 us pulse on 1-Wire bus to send a command to serial ID chip (write only), FPGA |

(*) These optional write commands cause generation of pulses of specific length onto dedicated backplane lines. Ccb_cmd[5..0], ccb_cmd_strobe, ccb_data[7..0], ccb_data_strobe lines remain inactive during these commands. These commands are independent from CSRB1[0].

4.1. CSRA1 (base + 00)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | Source of signals to custom backplane (FPGA if “0” and discrete logic interface if “1”). |
| 1 | R/W | Enable Read operations over I2C bus from TTCrx ASIC (active “0”) |
| 2 | R/W | I2C_SDA line (active “0”) |
| 3 | R/W | I2C_SCL line (active “0”) |
| 4 | R | I2C_SDA line (read only) |
| 5 | R/W | FPGA_TDI pin of the JTAG interface to FPGA/EPROM |
| 6 | R/W | FPGA_TMS pin of the JTAG interface to FPGA/EPROM |
| 7 | R/W | FPGA_TCK pin of the JTAG interface to FPGA/EPROM |
| 8 | R | FPGA_TDO pin of the JTAG interface from FPGA/EPROM (read only) |
| 9 | R/W | - |
| 10 | R/W | - |
| 11 | R/W | - |
| 12 | R/W | - |
| 13 | R/W | - |
| 14 | R/W | - |
| 15 | R/W | - |

  - CSRA2 (base + 02)

Write operation to CSRA2 (any data) causes unconditional “Hard Reset” of the  FPGA. Data format for read operations  is shown in a table below.

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | Mpc_cfg_done (peripheral crate) (active “0”) or MS_cfg_done (TF crate) (active “0”) |
| 1 | R | Alct_cfg_done_1 (peripheral crate) (active “0”) |
| 2 | R | Alct_cfg_done_2 (peripheral crate) (active “0”) |
| 3 | R | Alct_cfg_done_3 (peripheral crate) (active “0”) |
| 4 | R | Alct_cfg_done_4 (peripheral crate) (active “0”) or SP_cfg_done_3 (TF crate) (active “0”) |
| 5 | R | Alct_cfg_done_5 (peripheral crate) (active “0”) or SP_cfg_done_6 (TF crate) (active “0”) |
| 6 | R | Alct_cfg_done_6 (peripheral crate) (active “0”) or SP_cfg_done_7 (TF crate) (active “0”) |
| 7 | R | Alct_cfg_done_7 (peripheral crate) (active “0”) |
| 8 | R | Alct_cfg_done_8 (peripheral crate) (active “0”) or SP_cfg_done_10 (TF crate) (active “0”) |
| 9 | R | Alct_cfg_done_9 (peripheral crate) (active “0”) |
| 10 | R | Tmb_cfg_done_1 (peripheral crate) (active “0”) |
| 11 | R | Tmb_cfg_done_2 (peripheral crate) (active “0”) |
| 12 | R | Tmb_cfg_done_3 (peripheral crate) (active “0”) |
| 13 | R | Tmb_cfg_done_4 (peripheral crate) (active “0”) or SP_cfg_done_2 (TF crate) (active “0”) |
| 14 | R | Tmb_cfg_done_5 (peripheral crate) (active “0”) or SP_cfg_done_5 (TF crate) (active “0”) |
| 15 | R | Tmb_cfg_done_6 (peripheral crate) (active “0”) |

  - CSRA3  (base + 04)

Write operation to CSRA3 (any data) causes “Soft Reset” of the  FPGA logic, including reset of  CSRB11[10..8], reset of L1ACC counter, disable L1ACC counter, enable unconditional propagation of L1ACC from selected source(s) to custom backplane. Data format for read operations  is shown in a table below.

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | Tmb_cfg_done_7 (peripheral crate) (active “0”) or SP_cfg_done_8 (TF crate) (active “0”) |
| 1 | R | Tmb_cfg_done_8 (peripheral crate) (active “0”) |
| 2 | R | Tmb_cfg_done_9 (peripheral crate) (active “0”) or SP_cfg_done_11 (TF crate) (active “0”) |
| 3 | R | Dmb_cfg_done_1 (peripheral crate) (active “1”) |
| 4 | R | Dmb_cfg_done_2 (peripheral crate) (active “1”) |
| 5 | R | Dmb_cfg_done_3 (peripheral crate) (active “1”) or SP_cfg_done_1 (TF crate) (active “0”) |
| 6 | R | Dmb_cfg_done_4 (peripheral crate) (active “1”) or SP_cfg_done_4 (TF crate) (active “0”) |
| 7 | R | Dmb_cfg_done_5 (peripheral crate) (active “1”) |
| 8 | R | Dmb_cfg_done_6 (peripheral crate) (active “1”) |
| 9 | R | Dmb_cfg_done_7 (peripheral crate) (active “1”) or SP_cfg_done_9 (TF crate) (active “0”) |
| 10 | R | Dmb_cfg_done_8 (peripheral crate) (active “1”) |
| 11 | R | Dmb_cfg_done_9 (peripheral crate) (active “1”) or SP_cfg_done_12 (TF crate) (active “0”) |
| 12 | R | CCB’2004 FPGA configuration from EPROM done successfully (active”1”) |
| 13 | R | TTCrx_Ready status line (active “1” when “Ready”) |
| 14 | R | QPLL_lock output of the QPLL ASIC (active “0” when “Locked”) |
| 15 | R | All connected peripheral or TF boards (including CCB’2004 itself) have been configured from their EPROM’s successfully (active “0) |

4.4.  CSRB1 (base + 20)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | Defines the source of ccb_cmd[5..0], ccb_cmd_strobe, ccb_data[7..0], ccb_data_stobe signals distibuted to backplane when CSRA1[0]=0. The source is TTCrx if « 0 » and CSRB2 (for command bus) and CSRB3 (for data bus) if « 1 » |
| 1 | R/W | - |
| 2 | R/W | Mask L1ACC source from logical « OR » of three DMB_cfeb_calibrate[2..0] sources (disabled if « 1 » and enabled if « 0 ») |
| 3 | R/W | Mask L1ACC source from TTCrx (disabled if « 1 » and enabled if « 0 ») |
| 4 | R/W | Mask L1ACC from VME command (disabled if « 1 » and enabled if « 0 ») |
| 5 | R/W | Mask L1ACC from TMB_L1A_REQ backplane line (disabled if « 1 » and enabled if « 0 ») |
| 6 | R/W | Mask L1ACC from TMB_L1A_REL backplane line (disabled if « 1 » and enabled if « 0 ») |
| 7 | R/W | Mask L1ACC from the front panel (disabled if « 1 » and enabled if « 0 ») |
| 8 | R/W | Enable (when « 1 ») or Disable (when « 0 ») all inputs from the front panel |
| 9 | R/W | Mask generation of delayed ALCT_external_trigger signal from any L1A source (disabled if « 1 » and enabled if « 0 ») |
| 10 | R/W | Mask generation of delayed CLCT_external_trigger signal from any L1A source (disabled if « 1 » and enabled if « 0 ») |
| 11 | R/W | Mask generation of delayed Pretriggers and delayed L1ACC from any of ALCT_adb_sync_pulse sources (disabled if « 1 » and enabled if « 0 ») |
| 12 | R/W | Mask generation of delayed Pretriggers and delayed L1ACC from any of ALCT_adb_async_pulse sources (disabled if « 1 » and enabled if « 0 ») |
| 13 | R/W | If « 1 », then the CCB disables sending delayed L1ACC and both delayed Pretriggers to backplane after the first L1ACC has been translated to backplane. If « 0 », then L1ACC and Pretriggers are enabled unconditionally |
| 14 | R/W | When « 0 », the Tmb_l1A_Release signal enables propagation of delayed L1ACC and both delayed Pretriggers to custom backplane |
| 15 | R/W | When « 0 », the Dmb_l1A_Release signal enables propagation of delayed L1ACC and both delayed Pretriggers to custom backplane |

4.5.  CSRB2 (base + 22)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | BcntRes * |
| 1 | R/W | EvCntRes * |
| 2 | R/W | Data transmitted to ccb_cmd[0] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 3 | R/W | Data transmitted to ccb_cmd[1] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 4 | R/W | Data transmitted to ccb_cmd[2] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 5 | R/W | Data transmitted to ccb_cmd[3] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 6 | R/W | Data transmitted to ccb_cmd[4] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 7 | R/W | Data transmitted to ccb_cmd[5] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 8 | R/W | - |
| 9 | R/W | - |
| 10 | R/W | - |
| 11 | R/W | - |
| 12 | R/W | - |
| 13 | R/W | - |
| 14 | R/W | - |
| 15 | R/W | - |

* Ccb_cmd_strobe (25 ns pulse) is generated upon write data into CSRB2

4.6.  CSRB3 (base + 24)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | Data transmitted to ccb_data[0] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 1 | R/W | Data transmitted to ccb_data[1] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 2 | R/W | Data transmitted to ccb_data[2] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 3 | R/W | Data transmitted to ccb_data[3] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 4 | R/W | Data transmitted to ccb_data[4] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 5 | R/W | Data transmitted to ccb_data[5] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 6 | R/W | Data transmitted to ccb_data[6] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 7 | R/W | Data transmitted to ccb_data[8] when CSRA1[0]=0 and CSRB1[0]=1 * |
| 8 | R/W | - |
| 9 | R/W | - |
| 10 | R/W | - |
| 11 | R/W | - |
| 12 | R/W | - |
| 13 | R/W | - |
| 14 | R/W | - |
| 15 | R/W | - |

* Ccb_data_strobe (25 ns pulse) is generated upon write data into CSRB3

4.7.  CSRB4 (base + 26)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | - |
| 1 | R/W | - |
| 2 | R/W | - |
| 3 | R/W | - |
| 4 | R/W | - |
| 5 | R/W | - |
| 6 | R/W | - |
| 7 | R/W | - |
| 8 | R/W | - |
| 9 | R/W | - |
| 10 | R/W | - |
| 11 | R/W | - |
| 12 | R/W | - |
| 13 | R/W | - |
| 14 | R/W | - |
| 15 | R/W | - |

  - CSRB5 (base + 28)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane, LSB (*) |
| 1 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane (*) |
| 2 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane (*) |
| 3 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane (*) |
| 4 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane (*) |
| 5 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane (*) |
| 6 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane (*) |
| 7 | R/W | Delay of L1A (any of 7 sources) before distribution to backplane, MSB (*) |
| 8 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane, LSB (*) |
| 9 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane (*) |
| 10 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane (*) |
| 11 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane (*) |
| 12 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane (*) |
| 13 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane (*) |
| 14 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane (*) |
| 15 | R/W | Delay of ALCT and CLCT external triggers before distribution to backplane, MSB (*) |

(*) Delay step = 1BX (25 ns)

4.9.  CSRB6 (base + 2a)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | CCB_reserved[2] line (active « 1 ») |
| 1 | R/W | CCB_reserved[3] line (active « 1 ») |
| 2 | R/W | TMB_reserved[0] line (active « 1 ») |
| 3 | R/W | MPC_reserved[0] line (active « 1 ») |
| 4 | R/W | MPC_reserved[1] line (active « 1 ») |
| 5 | R/W | DMB_reserved[0] line (active « 1 ») |
| 6 | R/W | DMB_reserved[1] line (active « 1 ») |
| 7 | R/W | TMB_reserved_out[0] line (active « 1 ») |
| 8 | R/W | TMB_reserved_out[1] line (active « 1 ») |
| 9 | R/W | TMB_reserved_out[2] line (active « 1 ») |
| 10 | R/W | DMB_reserved_out[0] line (active « 1 ») |
| 11 | R/W | DMB_reserved_out[1] line (active « 1 ») |
| 12 | R/W | DMB_reserved_out[2] line (active « 1 ») |
| 13 | R/W | DMB_reserved_out[3] line (active « 1 ») |
| 14 | R/W | DMB_reserved_out[4] line (active « 1 ») |
| 15 | R/W | - |

4.10.  CSRB7 (base + 2c)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | QPLL_mode line. Set to « 1 » after power cycling. « 1 » is for the « 160MHz frequency multiplication mode » and « 0 » is for the « 120MHz frequency multiplication mode ». Effective for the QPLL when the S2-1 is « on ». |
| 1 | R/W | QPLL_reset line. Set to « 1 » after power cycling. Active « 0 ». Effective for the QPLL when the S2-4 is « on » and S2-5 is « off ». |
| 2 | R/W | QPLL_restart line. Set to « 1 » after power cycling. When « 1 », automatic restart of the PLL is enabled. A frequency calibration cycle will occur each time the PLL is detected to be unlocked or after a reset. When « 0 », automatic restart of the PLL is disabled. Effective for the QPLL when the S2-3 is « on ». |
| 3 | R/W | QPLL_excnt line. Set to « 0 » after power cycling. When « 0 », the VCXO centre frequency is set by the automatic frequency calibration procedure. When « 1 », the VCXO free running frequency is set by the input signals fsel[5:0]. |
| 4 | R/W | QPLL_fsel0 line. Set to « 0 » after power cycling. |
| 5 | R/W | QPLL_fsel1 line. Set to « 0 » after power cycling. |
| 6 | R/W | QPLL_fsel2 line. Set to « 0 » after power cycling. |
| 7 | R/W | QPLL_fsel3 line. Set to « 1 » after power cycling. |
| 8 | R/W | - |
| 9 | R/W | - |
| 10 | R/W | - |
| 11 | R/W | - |
| 12 | R/W | - |
| 13 | R/W | - |
| 14 | R/W | - |
| 15 | R/W | - |

4.11.  CSRB8 (base + 2e)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R/W | - |
| 1 | R/W | - |
| 2 | R/W | - |
| 3 | R/W | - |
| 4 | R/W | - |
| 5 | R/W | - |
| 6 | R/W | - |
| 7 | R/W | - |
| 8 | R/W | - |
| 9 | R/W | - |
| 10 | R/W | - |
| 11 | R/W | - |
| 12 | R/W | - |
| 13 | R/W | - |
| 14 | R/W | - |
| 15 | R/W | - |

4.12.  CSRB9 (base + 30)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | « 0 » indicates the « Presence pulse » from serial ID after the « Reset pulse » |
| 1 | R | Data bit from serial ID chip |
| 2 | R | Status of the initialization. When « 1 » after the « Reset pulse » , the CSRB9[0] is valid |
| 3 | R | Status of the read cycle. When « 1 » after the « Read pulse », the CSRB9[1] is valid |
| 4 | R | Status of the command cycle. When « 1 » after the « Write-one » or « Write-zero » command, the next command can be sent |
| 5 | R | « 0 » |
| 6 | R | « 0 » |
| 7 | R | « 0 » |
| 8 | R | « 0 » |
| 9 | R | « 0 » |
| 10 | R | « 0 » |
| 11 | R | « 0 » |
| 12 | R | « 0 » |
| 13 | R | « 0 » |
| 14 | R | « 0 » |
| 15 | R | « 0 » |

4.13.  CSRB10 (base + 32)

| Bit | Access | Function |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |
| 8 |  |  |
| 9 |  |  |
| 10 |  |  |
| 11 |  |  |
| 12 |  |  |
| 13 |  |  |
| 14 |  |  |
| 15 |  |  |

4.14.  CSRB11 (base + 34)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | DMB_reserved_in[0] line (« 1 » if backplane line is active) |
| 1 | R | DMB_reserved_in[1] line (« 1 » if backplane line is active) |
| 2 | R | DMB_reserved_in[2] line (« 1 » if backplane line is active) |
| 3 | R | TMB_reserved_in[0] line (« 1 » if backplane line is active) |
| 4 | R | TMB_reserved_in[1] line (« 1 » if backplane line is active) |
| 5 | R | TMB_reserved_in[2] line (« 1 » if backplane line is active) |
| 6 | R | TMB_reserved_in[3] line (« 1 » if backplane line is active) |
| 7 | R | TMB_reserved_in[4] line (« 1 » if backplane line is active) |
| 8 | R | TTC_SINERRSTR line (*) |
| 9 | R | TTC_DBERRSTR line (*) |
| 10 | R | QPLL_error line (*) |
| 11 | R | QPLL_lock line (active « 1 » when locked) |
| 12 | R | TTCrx_ready line (active « 1 » when ready) |
| 13 | R | « 0 » |
| 14 | R | « 0 » |
| 15 | R | « 0 » |

(*) – any error from these sources will set these bits “1”. They can  be reset by  sending “Reset FPGA logic” or “Reset CSRB11 error bits” commands.

4.15.  CSRB12 (base + 36)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | TTC_BCNT[0] line, latched on TTC_BCNTSTR strobe |
| 1 | R | TTC_BCNT[1] line, latched on TTC_BCNTSTR strobe |
| 2 | R | TTC_BCNT[2] line, latched on TTC_BCNTSTR strobe |
| 3 | R | TTC_BCNT[3] line, latched on TTC_BCNTSTR strobe |
| 4 | R | TTC_BCNT[4] line, latched on TTC_BCNTSTR strobe |
| 5 | R | TTC_BCNT[5] line, latched on TTC_BCNTSTR strobe |
| 6 | R | TTC_BCNT[6] line, latched on TTC_BCNTSTR strobe |
| 7 | R | TTC_BCNT[7] line, latched on TTC_BCNTSTR strobe |
| 8 | R | TTC_BCNT[8] line, latched on TTC_BCNTSTR strobe |
| 9 | R | TTC_BCNT[9] line, latched on TTC_BCNTSTR strobe |
| 10 | R | TTC_BCNT[10] line, latched on TTC_BCNTSTR strobe |
| 11 | R | TTC_BCNT[11] line, latched on TTC_BCNTSTR strobe |
| 12 | R | - |
| 13 | R | - |
| 14 | R | - |
| 15 | R | - |

  - CSRB13 (base + 38)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | TTC_BCNT[0] line, latched on TTC_EVCNTLSTR strobe |
| 1 | R | TTC_BCNT[1] line, latched on TTC_EVCNTLSTR strobe |
| 2 | R | TTC_BCNT[2] line, latched on TTC_EVCNTLSTR strobe |
| 3 | R | TTC_BCNT[3] line, latched on TTC_EVCNTLSTR strobe |
| 4 | R | TTC_BCNT[4] line, latched on TTC_EVCNTLSTR strobe |
| 5 | R | TTC_BCNT[5] line, latched on TTC_EVCNTLSTR strobe |
| 6 | R | TTC_BCNT[6] line, latched on TTC_EVCNTLSTR strobe |
| 7 | R | TTC_BCNT[7] line, latched on TTC_EVCNTLSTR strobe |
| 8 | R | TTC_BCNT[8] line, latched on TTC_EVCNTLSTR strobe |
| 9 | R | TTC_BCNT[9] line, latched on TTC_EVCNTLSTR strobe |
| 10 | R | TTC_BCNT[10] line, latched on TTC_EVCNTLSTR strobe |
| 11 | R | TTC_BCNT[11] line, latched on TTC_EVCNTLSTR strobe |
| 12 | R | - |
| 13 | R | - |
| 14 | R | - |
| 15 | R | - |

4.17.  CSRB14 (base + 3a)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | TTC_BCNT[0] line, latched on TTC_EVCNTHSTR strobe |
| 1 | R | TTC_BCNT[1] line, latched on TTC_EVCNTHSTR strobe |
| 2 | R | TTC_BCNT[2] line, latched on TTC_EVCNTHSTR strobe |
| 3 | R | TTC_BCNT[3] line, latched on TTC_EVCNTHSTR strobe |
| 4 | R | TTC_BCNT[4] line, latched on TTC_EVCNTHSTR strobe |
| 5 | R | TTC_BCNT[5] line, latched on TTC_EVCNTHSTR strobe |
| 6 | R | TTC_BCNT[6] line, latched on TTC_EVCNTHSTR strobe |
| 7 | R | TTC_BCNT[7] line, latched on TTC_EVCNTHSTR strobe |
| 8 | R | TTC_BCNT[8] line, latched on TTC_EVCNTHSTR strobe |
| 9 | R | TTC_BCNT[9] line, latched on TTC_EVCNTHSTR strobe |
| 10 | R | TTC_BCNT[10] line, latched on TTC_EVCNTHSTR strobe |
| 11 | R | TTC_BCNT[11] line, latched on TTC_EVCNTHSTR strobe |
| 12 | R | - |
| 13 | R | - |
| 14 | R | - |
| 15 | R | - |

4.18.  CSRB15 (base + 3c)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | - |
| 1 | R | - |
| 2 | R | TTC_BRCST[2] |
| 3 | R | TTC_BRCST[3] |
| 4 | R | TTC_BRCST[4] |
| 5 | R | TTC_BRCST[5] |
| 6 | R | TTC_BRCST[6] |
| 7 | R | TTC_BRCST[7] |
| 8 | R | - |
| 9 | R | - |
| 10 | R | TTC_DQ[0] |
| 11 | R | TTC_DQ[1] |
| 12 | R | TTC_DQ[2] |
| 13 | R | TTC_DQ[3] |
| 14 | R | - |
| 15 | R | - |

4.19.  CSRB16 (base + 3e)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | TTC_DOUT[0] line latched on TTC_DOUTSTR strobe |
| 1 | R | TTC_DOUT[1] line latched on TTC_DOUTSTR strobe |
| 2 | R | TTC_DOUT[2] line latched on TTC_DOUTSTR strobe |
| 3 | R | TTC_DOUT[3] line latched on TTC_DOUTSTR strobe |
| 4 | R | TTC_DOUT[4] line latched on TTC_DOUTSTR strobe |
| 5 | R | TTC_DOUT[5] line latched on TTC_DOUTSTR strobe |
| 6 | R | TTC_DOUT[6] line latched on TTC_DOUTSTR strobe |
| 7 | R | TTC_DOUT[7] line latched on TTC_DOUTSTR strobe |
| 8 | R | TTC_SUBAD[0] line latched on TTC_DOUTSTR strobe |
| 9 | R | TTC_SUBAD[1] line latched on TTC_DOUTSTR strobe |
| 10 | R | TTC_SUBAD[2] line latched on TTC_DOUTSTR strobe |
| 11 | R | TTC_SUBAD[3] line latched on TTC_DOUTSTR strobe |
| 12 | R | TTC_SUBAD[4] line latched on TTC_DOUTSTR strobe |
| 13 | R | TTC_SUBAD[5] line latched on TTC_DOUTSTR strobe |
| 14 | R | TTC_SUBAD[6] line latched on TTC_DOUTSTR strobe |
| 15 | R | TTC_SUBAD[7] line latched on TTC_DOUTSTR strobe |

4.20.  CSRB17 (base + 40)

| Bit | Access | Function |
| --- | --- | --- |
| 0 | R | Date of the current firmware version, Day (LSB) |
| 1 | R | Date of the current firmware version, Day |
| 2 | R | Date of the current firmware version, Day |
| 3 | R | Date of the current firmware version, Day |
| 4 | R | Date of the current firmware version, Day (MSB) |
| 5 | R | Date of the current firmware version, Month (LSB) |
| 6 | R | Date of the current firmware version, Month |
| 7 | R | Date of the current firmware version, Month |
| 8 | R | Date of the current firmware version, Month (MSB) |
| 9 | R | Date of the current firmware version, Year (LSB) * |
| 10 | R | Date of the current firmware version, Year * |
| 11 | R | Date of the current firmware version, Year * |
| 12 | R | Date of the current firmware version, Year (MSB) * |
| 13 | R | « 0 » |
| 14 | R | « 0 » |
| 15 | R | « 0 » |

* add to 2000 to obtain a year

4.21. CSRB18 (base + 42)

| Bit | Access | Function |
| --- | --- | --- |
| 0-7 | R | Represents the TTCrx hardwired ID on Data[7..0] lines. Valid after TTCrx reset command (write to base+5c then wait 65 us) |
| 15-8 | R | Represents the TTCrx hardwired ID on SubAdr[7..0] lines. Valid after TTCrx reset command (write to base+5c then wait 65 us) |

4.22. CSRB19 (base + 44)

| Bit | Access | Function |
| --- | --- | --- |
| 15-0 | R | 16 LCB bits of the 32-bit counter of BRCSTSTR1 pulse from the TTCrx ASIC |
|  | W | Resets the 32-bit counter to « 0 » (write any data) |

CSRB19 (base + 46)

| Bit | Access | Function |
| --- | --- | --- |
| 15-0 | R | 16 MSB bits of the 32-bit counter of BRCSTSTR1 pulses from the TTCrx ASIC |

4.23. CSRB21 (base + 48)

| Bit | Access | Function |
| --- | --- | --- |
| 15-0 | R | 16-bit counter of DOUTSTR pulses from the TTCrx ASIC |
|  | W | Resets the counter to « 0 » (write any data) |

4.24. CSRB22 (base + 4A)

| Bit | Access | Function |
| --- | --- | --- |
| 15-0 | R | 16-bit counter of QPLL_LOCK pulses from the QPLL ASIC |
|  | W | Resets the counter to « 0 » (write any data) |

4.25. CSRB23 (base + 4C)

| Bit | Access | Function |
| --- | --- | --- |
| 15-0 | R | 16-bit counter of TTC_READY pulses from the TTCrx ASIC |
|  | W | Resets the counter to « 0 » (write any data) |

4.26. CSRB24 (base + 4E)

| Bit | Access | Function |
| --- | --- | --- |
| 15-0 | R | 16-bit counter of QPLL_ERR pulses from the QPLL ASIC |
|  | W | Resets the counter to « 0 » (write any data) |

- JTAG Access to FPGA and EPROM

One Xilinx XC2V250-4FG456 FPGA that requires one XC18V02 EPROM is being used on CCB2004 board. Both chips can be accessed over JTAG bus; the EPROM is the first device in JTAG chain, and the FPGA is the second one. JTAG signals can be emulated using write and read operations directed to bits  CSRA1[8..5]. In addition to that, JTAG access from an external computer is possible using a Xilinx Parallel Cable IV.  An on-board jumper S10-1 defines which of these two options is activated.

- I2C  Programming

The I2C access to TTCrx ASIC  is supported by CSRA1[4..1] bits.  During read operations the  CSRA1[1] must be “0”. During write operations CSRA1[1] must be “1”.

For the TTCrx ASIC, all data transfers over the I2C bus are performed using only two registers: the I2C_pointer register and the I2C_data register. The I2C_pointer register is 5-bit wide and contains the address of the internal register as defined in Table 3 on page 13 in  [3]. Hence, each I2C access is performed in two steps:

- Write to register number in the I2C_pointer register

- Read or write the I2C_data register.

Each TTCrx ASIC occupies two addresses in the 7-bit I2C address space. The 7-bit I2C address is derived from the content of the ID_I2C <5..0> base address register as described in Table 9 based on [3]. ID_I2C<5..0> bits are defined by the jumpers on Dout<5..0>lines on the TTCrq mezzanine board.

Table 9: I2C address calculation

| I2C access register | Resulting 7-bit I2C address |
| --- | --- |
| I2C_pointer | ID_I2C<5..0> * 2 |
| I2C_data | ID_I2C<5..0> * 2 + 1 |

Note that the correct operation of the I2C bus requires the TTCrx to be locked to the TTC signal (“TTC_Ready”=1).

- Serial Number Access

There is a Silicon Serial Number DS2401 [8] chip that consists of a factory-lasered, 64-bit ROM that includes a unique 48-bit serial number, an 8-bit CRC, and an 8-bit Family Code (01h).  Data is transferred serially via the 1-Wire protocol, read and write least significant bit first. The protocol details and timing diagrams are given in [8]. An access to serial number chip consists of three phases: Initialization, ROM Function Command, and Read Data.

The Initialization sequence consists of a “Reset pulse” transmitted by the master followed by a “Presence pulse” transmitted by the DS2401. The “Presence pulse” lets the bus master  know that the DS2401 is on the bus and ready to operate. For the Initialization, the “Reset pulse” should be sent, then CSRB9[2] should be checked, and, when the CSRB9[2]=1, the CSRB9[0] should be checked. If CSRB9[0]=0 at this moment, that means that the “Presence pulse” was sent and the next step can be performed.

The ROM Function Command phase consists of sending a Read ROM command  [33h] or [0Fh] to DS2401. The first bit (“Write-one”) should be sent, then CSRB9[4] scanned, and, when CSRB9[4]=1, the next bit of command should be sent. Since all commands are 8-bit long,  eight write operations are necessary.

The Read Data phase consists of 64 read cycles. Each cycle starts with sending a “Read pulse”, then CSRB9[3] is scanned, and, when CSRB9[3]=1, the valid data bit should be received from  CSRB9[1]. Note the first data bit should be “1” and the next seven bits should be “0” (they represent the Family Code 01h). Bits 49-56 are also “0” and bits 57-64 represent the CRC code.

- On-board switches, jumpers and fuses

Switch S1 provides either +3.3V (S1-1 is “on” and S1-2 is “off”) or +5.0V (S1-1 is “off” and S1-2 is “on”) to all  components (except pin-preamplifier)  on TTCrq mezzanine. The jumper S1-2 should be installed. Pin-preamplifier is powered from +5V permanently.

Switch S2 provides control inputs for the TTCrx and QPLL ASICs. When S2-1 is “on”, the “mode” input of QPLL can be controlled from CSRB7[0]. When S2-1 is “off”, the mode=”1”  due to internal pull-up resistor. When S2-2 is “on”, the “externalControl” input of QPLL can be controlled from CSRB7[3]. When S2-2 is “off”, the externalControl=”1” due to internal pull-up resistor. When S2-3 is “on”, the “autoRestart” input of the QPLL can be controlled from CSRB7[2]. When S2-3 is “off”, the autoRestart=”1” due to internal pull-up resistor. When S2-4 is “on”, the “reset” input of QPLL can be controlled from CSRB7[1]. The S2-5 allows to connect (when “on”) a pull-up 1Kohm resistor to “reset” input of the QPLL. Either S2-4 or S2-5 should be “on”. When S2-6 is “on”, the  TTCrx ASIC will be reset by a 50 us long pulse generated on writing any data to address (base+5c), see Table 8.

8-position switches S3, S4, S6 and S9 are needed to provide a “crate_done” signal indicating that all peripheral boards and the CCB’2004 itself are in a correct initial state after power up and/or reconfiguration. When any of  these switches is set to “on”, the corresponding signal (see Table 10) will be participating in a decision. When “off”, the signal is not involved in a decision. Particularly, for all unplugged peripheral boards the corresponding switches should be set “off”. The resulting “crate_done” signal is active when all lines participating in a decision are active.

Table 10

| Switch | Source | Switch | Source | Switch | Source | Switch | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S3-1 | TMB1 | S4-1 | TMB9 | S6-1 | DMB8 | S9-1 | ALCT7 |
| S3-2 | TMB2 | S4-2 | DMB1 | S6-2 | DMB9 | S9-2 | ALCT8 |
| S3-3 | TMB3 | S4-3 | DMB2 | S6-3 | ALCT1 | S9-3 | ALCT9 |
| S3-4 | TMB4 | S4-4 | DMB3 | S6-4 | ALCT2 | S9-4 | MPC |
| S3-5 | TMB5 | S4-5 | DMB4 | S6-5 | ALCT3 | S9-5 | FPGA_done * |
| S3-6 | TMB6 | S4-6 | DMB5 | S6-6 | ALCT4 | S9-6 | TTCrx_ ready |
| S3-7 | TMB7 | S4-7 | DMB6 | S6-7 | ALCT5 | S9-7 | QPLL_lock |
| S3-8 | TMB8 | S4-8 | DMB7 | S6-8 | ALCT6 | S9-8 | FPGA lock* |

* Indicate that Xilinx FPGA on CCB’2004 board was configured from its EPROM and its internal DLL was locked successfully.

The S5 and S7 switches define the sources of the 40Mhz and 80Mhz clocks.

When S5-1 is “on”, the source of the main system clock is Clock40Des1 from TTCrx. When S5-2 is “on”, the source is CMOS40Mhz clock from the TTCrq mezzanine.

When S5-3 is “on”, the source is on-board oscillator (80.157Mhz divided by 2).

When S5-4 is “on” the clock is from QPLL ASIC after  LVDS-to-LVTTL translator residing on CCB2004 board.

Note that S5-2 and S5-4 provide essentially the same clock source, but  the clock from S5-2 is delayed ~2 ns in respect to S5-4 due to additional buffer on CCB’2004 board.

Only one S5 switch should be set “on”. The default setting is S5-4  “on”.

Switch S7  provides either 40Mhz or 80Mhz clock sources for selected slots (Table 6).

When S7-1 is “on”,  the source is 40Mhz from one of the sources specified  by S5.

The S7-2 should be used only when S7-1 is “off’. S7-2 defines the 80Mhz clock source from S7-3 or S7-4 to be distributed to backplane.

When S7-3 is “on”, the source of 80Mhz  clock is QPLL.

When S7-4 is “on”, the source of 80Mhz  clock is on-board oscillator.

Only one S7-3 or S7-4 should be set “on”.

The default  setting  is when both S7-1 and S7-3 are set “on” (all clocks are 40Mhz).

When S8-1 is “on”, the on-board oscillator turns off. S8-2 is not used.

Switch S10 is being used to configure/program the FPGA and EPROM over JTAG.

When S10-1 is “on”, both FPGA and EPROM are accessible over JTAG connector on the front panel. When S10-1 is “off”, they are accessible from VME using CSRA1[8..5].

S10-2 and S10-3 define the configuration mode of the FPGA (see Table 11 below). Master SelectMAP mode should be set by default.

Table 11

| S10-2 | S10-3 | Mode |
| --- | --- | --- |
| off | off | Slave Serial |
| on | off | Boundary Scan |
| off | on | Master SelectMAP |
| on | on | Not used |

S10-4, 5, 6 allow to change the order of EPROM and FPGA in a JTAG chain (Table 12).

Table 12

| S10-4 | S10-5 | S10-6 | JTAG chain |
| --- | --- | --- | --- |
| on | off | on | XC18V02VQ44C EPROM + XC2V250-FG456 FPGA |
| off | on | off | XC18V02VQ44C EPROM |
| All others |  |  | Not effective |

S10-7 controls the HSWAP_EN pin of the FPGA. If “on”, it activates the internal pull-up for user i/o in the device prior to configuration. By default, HSWAP_EN is tired “1” with internal pull-up resistor. S10-8 (when “on”) allows to reconfigure the FPGA from EPROM on “Hard_reset” commands (either dedicated TTC “CCB_Hard_Reset” or common TTC “Hard_Reset” (see Table 7) or VME write to CSRA2 register) .

The default settings are: S10-1 “off”, S10-2 “off”, S10-3 “on”, S10-4 “on”, S10-5 “off” and S10-6 “on”, S10-7 “off”, S10-8 “off”.

S11 should be used only for debugging purposes. It allows to select the geographical addresses GA[4..0] when VME64x backplane is not available. The default state is “off” for all S11 switches.

Jumpers JP1 and JP2 defines the mode of GTLP transmitters to custom backplane: either transparent (JP2 installed) or clocked (JP1 installed). Jumper JP3 (when installed) allows to turn all GTLP transmitters off (isolate them from backplane) when the FPGA is being configured. Jumper JP4 allows the FPGA to control the VME DACK line in addition to discrete logic interface. Jumper JP5, when installed, allows to reset the FPGA on common TTC “Hard_Reset” command (if S10-8 is “on”). If JP5 is not installed, but S10-8 is “on”, then only the dedicated TTC “CCB_Hard_Reset” command and VME write to CSRA2 will cause reloading of the FPGA. By default, JP2 and JP5 are installed, and JP1, JP3 and JP4 are not installed.

Fuse F4 must be installed at any time, it provides +5V power from VME backplane. Fuse F3 provides +3.3V power from the VME64x backplane, while F5 provides +3.3V from on-board voltage regulator U95. Only one (by default, F3) should be installed. Fuse F2 is needed to provide +1.5V for the FPGA core. It should be installed at any time. Fuse F1 is needed to provide +1.5V for on board GTLP receivers from custom backplane and should be installed as well.

- Front  Panel

The front panel of the CCB2004 is shown in Figure 4.

Figure 4: Front panel of the CCB2004

Connector P9 is intended for the Xilinx-compatible JTAG cable. Its pin assignment is shown in Table 13. Two 34-pin connectors (Tables 14-15) provide several status outputs for monitoring purposes. The 18 CLCT and ALCT status signals are provided from the peripheral custom backplane directly. The 40Mhz clock output is the same as selected by S5 (see Section 8). Four signals BC0, Cmd_strobe, Data_Strobe, L1_Reset (Table 14) are the ones that the CCB2004 translates to backplane in the “FPGA” mode. The fifth signal in Table 14, the L1A is a logical OR of 10 sources described in Section 3.3. These sources can be masked out with the CSRB1 and the result can be delayed with the CSRB5. These five outputs are valid in any (“Discrete Logic” or “FPGA”) mode of the CCB2004 operation. Texas Instruments SN75LVDS387 output LVDS  drivers are being used. An optional 100 Ohm termination networks  are provided  (socketed SIP packages). They are required for debugging purposes only and should be removed in case of cable connection to another board. In this case the LVDS lines should be terminated on a receiver end. Table 16 shows the LVDS inputs that are provided to the FPGA.

Table 13: Xilinx JTAG Connector (P9)

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | GND | 2 | +3.3V |
| 3 | GND | 4 | TMS |
| 5 | GND | 6 | TCK |
| 7 | GND | 8 | TDO (output from the FPGA/PROM) |
| 9 | GND | 10 | TDI (input for the FPGA/PROM) |
| 11 | GND | 12 | N/C |
| 13 | GND | 14 | N/C |

Table 14: Connector P4 (outputs)

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Clct_status[0]+ | 2 | Clct_status[0]- |
| 3 | Clct_status[1]+ | 4 | Clct_status[1]- |
| 5 | Clct_status[2]+ | 6 | Clct_status[2]- |
| 7 | Clct_status[3]+ | 8 | Clct_status[3]- |
| 9 | Clct_status[4]+ | 10 | Clct_status[4]- |
| 11 | Clct_status[5]+ | 12 | Clct_status[5]- |
| 13 | Clct_status[6]+ | 14 | Clct_status[6]- |
| 15 | Clct_status[7]+ | 16 | Clct_status[7]- |
| 17 | Clct_status[8]+ | 18 | Clct_status[8]- |
| 19 | Ccb_clock40+ | 20 | Ccb_clock40- |
| 21 | BC0+ | 22 | BC0- |
| 23 | L1A+ | 24 | L1A- |
| 25 | Cmd_Strobe+ | 26 | Cmd_Strobe- |
| 27 | Programmable Output_1+ (Data_Strobe+) | 28 | Programmable Output_1- (Data_Strobe-) |
| 29 | Programmable_Output_2+ (L1Reset+) | 30 | Programmable Output_2- (L1Reset-) |
| 31 |  | 32 |  |
| 33 |  | 34 |  |

Table 15: Connector  P6 (outputs)

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | Alct_status[0]+ | 2 | Alct_status[0]- |
| 3 | Alct_status[1]+ | 4 | Alct_status[1]- |
| 5 | Alct_status[2]+ | 6 | Alct_status[2]- |
| 7 | Alct_status[3]+ | 8 | Alct_status[3]- |
| 9 | Alct_status[4]+ | 10 | Alct_status[4]- |
| 11 | Alct_status[5]+ | 12 | Alct_status[5]- |
| 13 | Alct_status[6]+ | 14 | Alct_status[6]- |
| 15 | Alct_status[7]+ | 16 | Alct_status[7]- |
| 17 | Alct_status[8]+ | 18 | Alct_status[8]- |
| 19 |  | 20 |  |
| 21 |  | 22 |  |
| 23 |  | 24 |  |
| 25 |  | 26 |  |
| 27 |  | 28 |  |
| 29 |  | 30 |  |
| 31 |  | 32 |  |
| 33 |  | 34 |  |

Table 16: Connector P7 (LVDS inputs)

| Pin | Signal | Pin | Signal |
| --- | --- | --- | --- |
| 1 | External_l1accept+ | 2 | External_l1accept- |
| 3 | CLCT_external_trigger+ | 4 | CLCT_external_trigger- |
| 5 | ALCT_external_trigger+ | 6 | ALCT_external_trigger- |
| 7 |  | 8 |  |
| 9 |  | 10 |  |
| 11 |  | 12 |  |
| 13 |  | 14 |  |
| 15 |  | 16 |  |
| 17 |  | 18 |  |
| 19 |  | 20 |  |

There are 22 LEDs on the front panel:

- “+3.3” from VME64x backplane (if F3 is installed) or from on-board voltage regulator U95 (if F5 is installed) (green, D23)

- “+5.0” from VME backplane (green, D19)

- “+1.5”  for FPGA core, from on-board voltage regulator U93 (green, D24)

- “+1.5B”  for GTLP terminators from custom backplane  powers (green, D25)

- “L1A”  L1ACCEPT (red, with one-shot, indicates that the L1ACC was sent to the backplane, in the “Discrete Logic” mode originates only from the TTCrq mezzanine; in the “FPGA” mode may be produced from several sources;  D1)

- “BC0”  BC0 (red, with one-shot, if BC0 signal was sent to backplane, D3)

- “HRESET”  Hard_Reset (any Hard_Reset except the CCB_Hard_reset) that has been sent to backplane) (red, with one-shot, D5)

- “CCBHR” CCB_Hard_Reset (red, with one-shot, D7)

- “L1RES” L1 Reset (red, with one-shot, D10)

- “EVCRES” Event Counter Reset (red, with one-shot, D6)

- “BCRES” Bunch Counter Reset (red, with one-shot, D8)

- “CMDSTR” Command Strobe  (red, with one-shot, D9)

- “DATSTR” Data Strobe (red, with one-shot, D11)

- “DACK” indicates VME access to CCB2004 board (yellow, with one-shot, D12)

- “TTCRDY” TTCrx_Ready indicates that the TTCrx is in normal operation mode (green)

- “JTAG”  indicates JTAG access from VME to FPGA using  CSRA1[8..5] bits (yellow, D2)

- “I2C”  indicates access to TTCrx ASIC over I2C bus using CSRA1[4..1] bits (yellow, D4)

- “DISLOG” Discrete_Logic indicates that commands are sent from the TTC system to custom backplane through the discrete logic interface (when CSRA1[0]=1) (green, D18)

- “DONE” configuration of the Xilinx FPGA from its EPROM was done successfully (green, D14).

- “CRDONE” Crate_Done configuration of all connected  boards installed in a crate from their EPROM’s was done successfully AND TTCrq board is READY AND an FPGA was configured from its EPROM successfully AND QPLL on a TTCrq board was locked (green, D15)

- “QLOCK” QPLL_Locked status (green, D16).

- “CLK40” CLOCK indicates that the main 40Mhz clock from the selected source defined by S5 is provided for FPGA and the DLL in FPGA is locked (green, counter output running at ~10Hz, D17).

# References

[1]. EMU Peripheral Crate  Specification. UCLA High Energy Physics. Version 3.3. February 9, 2001. Available at: ftp://www-collider.physics.ucla.edu/pub/cms/peripheral_crate/documentation/

[2]. B.G.Taylor. Timing, Trigger and Control (TTC) System for LHC Detectors. CERN/ECP http://www.cern.ch/TTC/intro.html

[3].  J.Christiansen, A.Marchioro, P.Moreira and T.Toifl. TTCrx Reference Manual. August 2005. Version 3.10. CERN-ECP/MIC,  Geneva, Switzerland. Available at

http://bonner-ntserver.rice.edu/cms/TTCrx_manual3.10.pdf

[4]. Clock and Control Board for the CSC Peripheral and Track Finder Electronics. Specification. Available at http://bonner-ntserver.rice.edu/CMS/CCB'2001_20.pdf

[5]. http://proj-qpll.web.cern.ch/proj-qpll/

[6]. Alex Madorsky. University of Florida. Peripheral backplane pin assignment.      Available at: http://www.phys.ufl.edu/~madorsky/backplane/PinAssignment.doc

[7]. CSC Track Finder Crate Specification.    Draft 1/09/2003.   Available at: http://www.phys.ufl.edu/~madorsky/TrackFinder/TF_backplane_v4.doc

[8]. DS2401 Silicon Serial Number Specification. Available at http://pdfserv.maxim-ic.com/en/ds/DS2401.pdf

History

10/13/2003:  Initial release

10/15/2003:  Change in  decoding scheme: use data[7..2] instead of data[5..0] (Table 7).

11/12/2003: Tables 4B and 5B were added.

12/11/2003: QPLL_locked signal was added to custom backplane (former ccb_reserved[1]).

12/16/2003: Table 11 was added.

01/16/2004: Table 3 was renamed to Table 3A. Table 3B was added.

01/20/2004: Input connector (Table 12) was changed to 20-pin.

02/27/2004: Table 8 (VME addresses) and CSRB registers were added.

03/12/2004: Section 7 (Serial Number Access) was added.

03/19/2004: Figure 2 was added.

03/22/2004: Correction in Table 12, sections 4.5 and 4.6.

03/31/2004: Figure 1 was updated.

08/13/2004: Fixes in Section 8.

10/13/2004: CSRB18 was added. “Reset TTCrx” command was added.

11/10/2004: Corrections is Tables 4B, 5A, 5B.

11/17/2004: Change from version 2.0 to version 3.0 (production board)

Update of 4.1, 4.2, 4.3.

12/24/2004: Changes in section 4.18.

04/06/2005: Section 2 was expanded. Update of Section 8.

04/12/2005: Section 8 was updated.

07/28/2006: Minor corrections throughout the text.

09/01/2006: Changes in sections 3.3 (DMB_cfeb_calibrate[2..0] pulses were added to a list of the L1ACC sources) and 4.4 (bit CSRB1[2] was defined). Changes in register format, sections 4.15, 4.16, 4.17, 4.18.

06/27/2007: Command OC0 (Orbit Counter Reset) was added (Table 7).

02/15/2008: Registers CSRB19, CSRB20 and CSRB21 were added.

04/02/2008: Minor addition in Section 9.

08/21/2008: Registers CSRB22, CSRB23 and CSRB24 were added. Section 4.10 was updated.

04/30/2009: Figure 4 and new Table (now Table 13) were added to Section 9.

10/21/2010: CSRB20 was removed. CSRB19 was expanded to 32 bits.